| - COMPETENCY-BASED CURRICULUM |  |
| --- | --- |

| Sector: ###### AUTOMOTIVE |  |
| --- | --- |
| Qualification: Automotive Servicing NC III (LPG Retrofitting and Re-powering) |  |
|  | Technical Education and Skills Development Authority East Service Road, South Superhighway, Taguig City, Metro Manila |

# TABLE OF CONTENTS

Page

A.	COURSE DESIGN	1-7

B.	MODULES OF INSTRUCTION	8-125

- Basic Competencies	8

  - Leading workplace communication	9-12

  - Leading small team	13-17

  - Developing and practicing negotiation skills	18-21

  - Identifying/determining fundamental cause of problem	22-25

  - Using mathematical concepts and techniques	26-29

  - Using relevant technologies	30-33

- Common Competencies	34

  - Applying appropriate sealant/adhesive	35-39

  - Moving and position vehicle	40-43

  - Performing mensuration and calculation	43-45

  - Reading, interpreting and applying specification and manual	46-50

  - Using and applying lubricant/coolant	51-54

  - Performing shop maintenance	55-59

- Core Competencies	60

  - Testing and repairing electrical security system/ components	61-63

  - Servicing electronic engine management system	64-66

  - Inspecting and analyzing engine failure	67-74

  - Inspecting and diagnosing automatic transmission	75-77

  - Pulling out, disassembling and checking automatic transmission	78-82

  - Assembling and testing automatic transmission	83-87

  - Performing maintenance service check-up and repair air-conditioning system	88-90

  - Installing air-conditioning systems	91-97

  - Servicing aircon compressor and associated components	98-106

  - Installing LPG conversion kit	107-110

  - Testing and adjusting LPG calibration	111-115

  - Servicing auto LPG system	116-120

  - Removing and replacing automotive engine and engine-related systems	121-125

# COURSE DESIGN

# COURSE TITLE	:	AUTOMOTIVE SERVICING NC III

NOMINAL DURATION	:	440 hours

(100 hours) for LPG Fuel Conversion

COURSE DESCRIPTION	:

This course is designed to enhance the knowledge, skills and attitudes of an individual in the field of automotive servicing in accordance with industry standards. It covers core competencies such as; test and repair electrical security system/components; service and repair electronic engine management system; overhaul engines and associated components; service  automatic transmission; perform maintenance service check-up and repair to air-conditioning system; and service and repair aircon compressor and associated components. In addition, LPG-Fuel Conversion competencies were included: install LPG conversion kit, test and adjust LPG calibration, service LPG system, and replace diesel-fuel with LPG-fuel engine

ENTRY REQUIREMENTS:

Student/trainee should possess the following requirements:

- can communicate both oral and written

- physically and mentally fit;

- with good moral character; and

- can perform basic mathematical computation.

For LPG Fuel conversion:

- Holder of Automotive Servicing NC II or its equivalent

COURSE STRUCTURE:

# BASIC COMPETENCIES

(20 hours)

| # Units of Competency | # Module Title | # Learning Outcomes | # Nominal Duration |
| --- | --- | --- | --- |
| # Lead workplace communication | # Leading workplace communication | - Communicate information about workplace processes. - Lead workplace discussions - Identify and communicate issues arising in the workplace | # 3 hours |
| # Lead small team | # 2.1 Leading small team | - Provide team leadership. - Assign responsibilities among members. - Set performance expectation for team members - Supervise team performance | # 3 hours |
| # Develop and practice negotiation skills | # 3.1 Developing and practicing negotiation skills | - Identify relevant information in planning negotiations - Participate in negotiations - Document areas for agreement | # 4 hours |
| - Solve problem related to work activities | # 4.1 Identifying/ determining fundamental cause of problem | - Explain the analytical techniques. - Identify the problem. - Determine the possible cause/s of the problem. | # 4 hours |
| - Use mathematical concepts and techniques | # 5.1 Using mathematical concepts and techniques | - Identify mathematical tools and techniques to solve problem - Apply mathematical procedures/solution - Analyze results | # 3 hours |
| - Use relevant technologies | ##### 6.1 Using relevant technologies | - Identify appropriate technology - Apply relevant technology - Maintain/enhance relevant technology | # 3 hours |

# COMMON COMPETENCIES

(20 hours)

| # Units of Competency | # Module Title | # Learning Outcomes | # Nominal Duration |
| --- | --- | --- | --- |
| - Apply appropriate sealant/ adhesive | - Applying appropriate sealant/ adhesive | - Identify appropriate sealant/adhesive - Prepare surface for sealant/adhesive application - Store unused and dispose used sealant/adhesive | # 3 hours |
| - Move and position vehicle | 2.1 Moving and positioning vehicle | - Prepare vehicle for driving - Move and position vehicle - Check the vehicle | # 3 hours |
| #### 3. Perform mensuration and calculation | 3.1 Performing mensuration and calculation | - Select measuring instrument and carry out measurement and calculations - Maintain measuring instrument | # 4 hours |
| - Read, interpret and apply specifications and manual | - Reading, interpreting and applying specifications and manual | - Identify/access manuals and interpret data and specification - Apply information accessed in manual - Store manual | # 4 hours |
| - Use and apply lubricant/ coolant | - Using and applying lubricant/ coolant | - Identify the type of lubricant/coolant - Use and apply lubricant/coolant | # 3 hours |
| - Perform shop maintenance | - Performing shop maintenance | - Inspect/clean tools and shop equipment - Store/arrange tools and shop equipment - Dispose waste/used lubricant - Report damaged tools/equipment | # 3 hours |

# CORE COMPETENCIES

(500 hours)

| # Units of Competency | # Module Title | # Learning Outcomes | # Nominal Duration |
| --- | --- | --- | --- |
| - Test and repair electrical security system/ components | 1.1 Testing and repairing electrical security system/ components | - Install electrical security system/components - Test and repair anti-theft system and components | # 40 hours |
| 2. Service electronic engine management system | 2.1 Servicing electronic engine management system | - Check electronic engine management system and/or associated components - Service electronic engine management associated parts | # 40 hours |
| - Overhaul engine and associated components | 3.1 Inspecting and analyzing engine failure | - Analyze engine failure - Pull down the engine - Disassemble engine parts and components - Inspect engine parts and components - Perform fitting of engine parts - Assemble engine parts and components - Mount and test run engine | # 80 hours |
| - Service automatic transmission | 4.1 Inspecting and diagnosing automatic transmission | - Explain the principles and components of automatic transmission - Diagnose automatic transmission | # 28 hours |
|  | 4.2 Pulling-out, disassembling, and checking automatic transmission. | - Dismount automatic transmission assembly - Disassemble automatic transmission - Conduct automatic transmission parts analysis - Replace automatic transmission parts | # 40 hours |
|  | 4.3 Assembling and testing automatic transmission | - Assemble automatic transmission - Mount install automatic transmission assembly - Test automatic transmission - Road test automatic transmission | # 40 hours |
| 5. Perform maintenance service check-up and repair to air-conditioning system | 5.1 Performing maintenance servicing check-up and repair to air-conditioning system | - Diagnose air-conditioning system - Service auto air conditioning system components | # 40 hours |
| 6. Install auto air-conditioning system | 6.1 Installing auto air-conditioning system | - Fabricate brackets and air-conditioning piping - Install air-conditioning systems - Leak test air-conditioning system - Evacuate air-conditioning system - Charge air-conditioning system - Test air-conditioning systems operation | # 40 hours |
| 7. Service auto air-conditioning compressor and associated components | 7.1 Servicing auto air-conditioning compressor and associated components | - Diagnose air-conditioning compressor. - Pull down compressor assembly - Disassemble compressor - Perform leak test of air-conditioning compressor - Assemble compressor - Install air-conditioning compressor - Evacuate and recharge air-conditioning system - Conduct compressor performance test. | # 52 hours |
| 8. Install LPG conversion kit | 8.1 Installing LPG conversion kit | - Perform vehicle test - Mount and install conversion kit - Install piping/hoses and safety features of the kit | # 28 hours |
| 9. Test and adjust LPG calibration | 9.1 Testing and adjusting LPG calibration | - Test electrical connections - Adjust ignition system - Adjust/tune-up vaporizer - Check/test LPG fuel system | # 26 hours |
| 10. Service auto LPG system | 10.1 Servicing auto LPG system | - Service electrical system - Service ignition components - Service vaporizer/mixer - Service LPG fuel system | # 16 hours |
| 11. Remove and replace automotive engine and engine-related systems | 11.1 Removing and replacing automotive engine and engine-related systems | - Prepare for work - Remove engine and/or related systems - Replace engine and/or related systems - Clean up work area | # 30 hours |

# ASSESSMENT METHODS:

- Written examination

- Demonstration of practical skills

- Direct observation with questioning

- Interview

COURSE DELIVERY:

- Self-paced instruction

- Demonstration

- Lecture

- Discussion

- Dual training

RESOURCES:

| - Equipment - Vehicle - Engine - Hydraulic jack/lift - Differential and front axle - Transmission/manual/automatic - Tools /accessories/supplies - Set of box wrench - Set of socket wrench - Set pliers - Set of screw driver - Wire strippers - Set of mechanic hammer - Apron/ goggles/ gloves - Engine oils - Grease - Sealant/adhesive - Hydraulic oils/gear oil - Wheel wedges - Torque wrench - Feeler gauge - Automatic transmission fluid | - Testing instruments - Multi-meter - Dial gauge - Bore gauge - Micrometer caliper - Vacuum gauge - Pressure gauge - Tachometer - Ignition timing light - Growler tester - Battery load tester - Training materials - Reference books - Manuals - Catalogs - Brochures - Modules/LEs - CDs/Video tapes - LPG-Fuel Conversion - LPG-fuel conversion kit - LPG fuel tank - Valves (solenoid, safety, emergency shut-off, etc.) - Fuel piping lines and fittings |
| --- | --- |

# TRAINER’S QUALIFICATIONS:

- Must be a holder of National Certificate (NC) IV for Automotive Servicing or its equivalent

- Must have undergone training on Training Methodology

- Must be physically and mentally able

- *Must have at least 2 years job/industry experience

- Must be a civil service eligible or appropriate professional license issued by the Professional Regulatory Commission (for government position)

* Optional. Only when required by the hiring institution.

MODULES OF INSTRUCTION

BASIC COMPETENCIES

AUTOMOTIVE SERVICING NC III

BASIC COMPETENCY	:	COMMUNICATIONS

UNIT OF COMPETENCY	:	LEAD WORKPLACE COMMUNICATION

MODULE TITLE	:	LEADING WORKPLACE COMMUNICATION

MODULE DESCRIPTOR	: 	This module covers the knowledge, skills and attitudes required to prepare different reports required in the workplace.

SUGGESTED DURATION	:	3 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module the students/trainees will be able to:

LO1.	Communicate information about workplace processes.

LO2.	Lead workplace discussions.

LO3.	Identify and communicate issues arising in the workplace.

LO1.	COMMUNICATE INFORMATION ABOUT WORKPLACE PROCESSES

# ASSESSMENT CRITERIA:

- Appropriate communication method is selected.

- Multiple operations involving several topic areas are communicated.

- Questions are used to gain extra information.

- Correct sources of information are identified.

- Information is selected and sequenced correctly when required.

- Verbal and written reporting is maintained in both familiar and unfamiliar situations.

CONTENTS:

- Method of communication

- Communication skills

- Communication tools

- Questioning techniques

CONDITIONS:

The students/trainees must be provided with the following:

- Simulated workplace environment

- Communication tools

- Variety of information’s

METHODOLOGIES:

- Discussion

- Role play

- Brainstorming

ASSESSMENT METHODS:

- Direct observation

- Interview

LO2.	LEAD WORKPLACE DISCUSSIONS

# ASSESSMENT CRITERIA:

- Responses to workplace issues are sought.

- Response to workplace issues are provided when sought.

- Constructive contributions are made to workplace discussion on such issues as production, quality and safety.

- Goals and aims of actions under taken in the workplace are communicated.

CONTENTS:

- Method/techniques of discussion

- How to lead discussion

- How to solicit response

CONDITIONS:

The students/trainees must be provided with the following:

- Simulated workplace environment

- Communication tools

- Variety of information’s

METHODOLOGIES:

- Discussion

- Role play

- Brainstorming

ASSESSMENT METHODS:

- Direct observation

- Interview

LO3.	IDENTIFY AND COMMUNICATE ISSUES ARISING IN THE WORKPLACE

# ASSESSMENT CRITERIA:

- Issues and problems are identified as they arise.

- Information regarding problems and issues are organized coherently to ensure clear and effective communication.

- Dialog is initiated with appropriate personnel.

- Communication problems and issues are addressed as they arise.

CONTENTS:

- Identify problems and issues

- Organizing information on  problem and issues

- Relating problems and issues

- Communication barriers affecting workplace discussions.

CONDITIONS:

The students/trainees must be provided with the following:

- Simulated workplace environment

- Communication tools

- Variety of information’s

METHODOLOGIES:

- Discussion

- Role play

- Brainstorming

ASSESSMENT METHODS:

- Direct observation

- Interview

BASIC COMPETENCY	:	TEAM WORK

UNIT OF COMPETENCY	:	LEAD SMALL TEAM

MODULE TITLE	:	LEADING SMALL TEAM

MODULE DESCRIPTOR	: 	This module covers the knowledge, skills and attitudes required to lead small team including setting and maintaining team and individual performance standard.

SUGGESTED DURATION	:	3 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module the students/trainees will be able to:

LO1.	Provide team leadership

LO2.	Assign responsibilities among members

LO3.	Set performance expectation for team members

LO4.	Supervise team performance

LO1.	PROVIDE TEAM LEADERSHIP

# ASSESSMENT CRITERIA:

- Work requirements are identified and prescribed to members.

- Reasons for instructions and requirements are properly disseminated to team members.

- Team members questions, problems, concerns are recognized, discussed and dealt accordingly.

CONTENTS:

- Communication skills required for leading small team

- Skills and techniques in promoting  team building

- Negotiating skills

- Up to date dissemination of instruction and requirements to members.

- Art of listening and treating individual team members concern

CONDITIONS:

The students/trainees must be provided with the following:

- Learning materials

- team building manual

- catalogs

- brochures

- Simulated team

METHODOLOGIES:

- Traditional /lecture

- Demonstration

- Case studies

ASSESSMENT METHODS:

- Written examination

- Direct observation

LO2.	ASSIGN RESPONSIBILITIES AMONG MEMBERS

ASSESSMENT CRITERIA:

- Duties and responsibilities are allocated in respect to the skills, knowledge and attitudes of every team member.

- Duties are allocated having regard to individual preference, domestic and personal considerations.

- Duties and responsibilities of each member are properly identified and defined.

CONTENTS:

- Duties and responsibilities of each team member

- Skills in identifying individual skills ,knowledge  and attitude as basis for allocating responsibilities

- Knowledge in identifying each team member duties and responsibilities

CONDITIONS:

The students/trainees must be provided with the following:

- Learning materials

- relevant legal requirements

- manuals

METHODOLOGIES:

- Traditional /lecture

- Demonstration

ASSESSMENT METHODS:

- Written examination

- Direct observation

LO3.	SET PERFORMANCE EXPECTATION FOR TEAM MEMBERS

# ASSESSMENT CRITERIA:

- Performance expectations are established based on client needs and according to assigned requirements.

- Performance expectations are based on individual team member’s duties and responsibilities.

- Performance expectations are discussed and disseminated to individual team member.

CONTENTS:

- Knowledge and skills in setting individual performance target/expectation

- Team members duties  and responsibilities

- Employee policies and procedures

- Defining performance expectations criteria

CONDITIONS:

The students/trainees must be provided with the following:

- Performance expectation worksheet

- Relevant legal requirements

METHODOLOGIES:

- Traditional /lecture

- Demonstration

- Case studies

ASSESSMENT METHODS:

- Written examination

- Direct observation

LO4.	SUPERVISE TEAM PERFORMANCE

# ASSESSMENT CRITERIA:

- Team member’s performance is monitored in respect to the defined performance criteria.

- Team members are provided with feedback, positive support and advice on strategies to overcome any difficulties.

- Team members are informed of any changes in the priority allocated to assignment or task.

- Communication follow-up is provided on all issues affecting the team.

CONTENTS:

- Knowledge and skills in monitoring team member performance

- Monitoring team operation to ensure client needs  and satisfaction

- Methods of monitoring performance

- Informal/formal counseling skills

CONDITIONS:

The students/trainees must be provided with the following:

- Performance expectation worksheet

- Relevant legal requirements

METHODOLOGIES:

- Traditional /lecture

- Demonstration

- Modular

ASSESSMENT METHODS:

- Written examination

- Direct observation

UNIT OF COMPETENCY	:	DEVELOP AND PRACTICE NEGOTIATION SKILLS

MODULE TITLE	:	Developing and Practicing Negotiation Skills

MODULE DESCRIPTOR	: 	This module covers the skills, knowledge and attitudes required to collect information in order to plan and participate in the negotiation.

NOMINAL DURATION	:    4 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module the students/ trainees will be able to:

# LO1.	Identify relevant information in planning negotiations

LO2.	Participate in negotiations

LO3.	Document areas for agreement

# LO1.	Identify relevant information in planning negotiations

ASSESSMENT CRITERIA:

- Information in preparation for negotiation is identified and included in the plan.

- Information on creating non verbal environments for positive negotiations is identified and included in the plan.

- Information on different questioning techniques is identified and included in the plan.

CONTENTS:

- Background information on other parties to the negotiation

- Observing differences between content and process

- Identifying bargaining information

- Applying strategies to manage process

- Applying steps in negotiating process

- Strategies to manage conflict

- Steps in negotiating process

CONDITIONS:

The students/ trainees must be provided with the following:

- Pertinent documents

- Simulated workplace

- Paper and pencil

- Calculator

- Hands out

METHODOLOGIES:

- Lecturette

- Role playing

- practical exercises

ASSESSMENT METHODS:

- Written test/ questioning

- Demonstration

# LO2.	Participate in Negotiations

ASSESSMENT CRITERIA:

- Criteria for successful outcome are agreed upon by all parties.

- Desired outcome of all parties are considered.

- Appropriate language is used throughout the negotiation.

CONTENTS:

- Decision making and conflict resolution strategies procedures

- Problem solving strategies on how to deal with unexpected questions and attitudes during negotiation

- Background information on other parties to the negotiation

- Observing differences between content and process

CONDITIONS:

The students/ trainees must be provided with the following:

- Pertinent documents

- Simulated workplace

- Supplies

- Paper and pencil

- Calculator

- Hands out

METHODOLOGIES:

- Lecturette

- Role playing

- practical exercises

ASSESSMENT METHODS:

- Written test/ questioning

- Demonstration

# LO3.	Document areas for negotiation

ASSESSMENT CRITERIA:

- The issues and processes are documented and agreed upon by all parties.

- Possible solutions are discussed and their viability assessed.

- Areas for agreement are confirmed and recorded.

- Follow-up action is agreed upon by all parties.

CONTENTS:

- Procedure in documenting negotiations

- Managing information

- Filing documents

CONDITIONS:

The students/ trainees must be provided with the following:

- Pertinent documents

- Simulated workplace

- Supplies

- Paper and pencil

- Calculator

- Hands out

METHODOLOGIES:

- Lecturette

- Role playing

- practical exercises

ASSESSMENT METHODS:

- Written test/ questioning

- Demonstration

BASIC COMPETENCY	:	PROBLEM SOLVING

UNIT OF COMPETENCY	:	SOLVE PROBLEMS RELATED TO WORK ACTIVITIES

MODULE TITLE	:	IDENTIFYING / DETERMINING

FUNDAMENTAL CAUSE OF PROBLEM

MODULE DESCRIPTOR	: 	This module expresses the competency required to apply problem solving techniques to identify/determine fundamental cause problem.

SUGGESTED DURATION	:	4 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module the students/trainees will be able to:

LO1.	Explain the analytical techniques

LO2.	Identify the problem

LO3.	Determine the possible cause/s of the problem

LO1.	EXPLAIN THE ANALYTICAL TECHNIQUES

# ASSESSMENT CRITERIA:

- Importance and application of analytical techniques are explained.

- Analytical techniques such as brainstorming, cause and effects diagrams, PARETO analysis, SWOT analysis, GANT chart, PERT CPM & graphs, and scatter grams are defined.

CONTENTS:

- Observation, investigation & analytical techniques

- Brainstorming

- Cause and effect diagrams

- PARETO analysis

- SWOT analysis

- GANT chart

- PERT CPM & graph

- SCATTERGRAMS

CONDITIONS:

The students/trainees must be provided with the following:

- Coursewares

- Learning materials/guides

- Computer

- OHP

METHODOLOGIES:

- Direct observation

- Simulation /role playing

- Case studies

ASSESSMENT METHODS:

- Written

- Practical/performance test

LO2.	IDENTIFY THE PROBLEM

# ASSESSMENT CRITERIA:

- Variances are identified from normal operating parameters and product quality.

- Extent, cause, and nature of the problem are defined based on observation, investigation and analytical techniques.

- Problems are clearly stated and specified.

CONTENTS:

- Normal operating parameters & product quality

- Identifying & clarifying the nature of problem

- Application of analytical techniques

CONDITIONS:

The students/trainees must be provided with the following:

- Coursewares

- Learning materials/guides

- Computer

- OHP

METHODOLOGIES:

- Direct observation

- Simulation /role playing

- Case studies

ASSESSMENT METHODS:

- Written

- Practical/performance test

LO3.	DETERMINE THE POSSIBLE CAUSE/S OF THE PROBLEM

# ASSESSMENT CRITERIA:

- Possible causes of problems are identified based on experience & the use of problem solving tools/analytical techniques.

- Possible cause statements are developed.

- Fundamental causes are explained.

CONTENTS:

- Non-routine process and quality problems

- Teamwork and work allocation problem

- Safety and emergency situations and incidents

CONDITIONS:

The students/trainees must be provided with the following:

- Coursewares

- Learning materials/guides

- Computer

- OHP

METHODOLOGIES:

- Direct observation

- Simulation /role playing

- Case studies

ASSESSMENT METHODS:

- Written

- Practical/performance test

UNIT OF COMPETENCY	:	USE MATHEMATICAL CONCEPTS AND TECHNIQUES

MODULE TITLE	:	USING MATHEMATICAL CONCEPTS AND

TECHNIQUES

MODULE DESCRIPTOR	:	This module covers the knowledge, skills and attitudes required in the application of mathematical concepts and techniques.

NOMINAL DURATION	:	3 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of the module, the learner/students must be able to:

LO1.	Identify mathematical tools and techniques to solve problems

LO2.	Apply mathematical procedure/solution

LO3. 	Analyze results

LO1.	Identify mathematical tools and techniques to solve problems

# ASSESSMENT CRITERIA:

- Problem areas based on given condition are identified.

- Mathematical techniques based on the given problem are selected.

CONTENTS:

- Four Fundamental Operations

- Steps in solving a problem

- Standard formulas

- Conversion

- Measurement

CONDITION:

The students/learners must be provided with the following:

- Manuals

- Hand-outs

- Problem set

- Conversion table

- Table of formulas

- Measuring tools

METHODOLOGIES:

- Lecturette

- Self-paced instruction

- Group discussion

ASSESSMENT METHODS:

- Written

- Demonstration

LO2.	Apply mathematical procedure/solution

# ASSESSMENT CRITERIA:

- Mathematical techniques based on the problem identified are applied.

- Mathematical computations are performed to the level of accuracy required for the problem.

- Results of mathematical computation based on job requirements is determined and verified.

CONTENTS:

- Problem-based questions

- Estimation

- Use of mathematical tools and standard formulas

- Mathematical techniques

CONDITION:

The students/learners must be provided with the following:

- Manuals

- Hand-outs

- Calculator

- Measuring tools/devices

- Case problems

METHODOLOGIES:

- Lecturette

- Self-paced instruction

- Group discussion

- Practical Work Approach

ASSESSMENT METHODS:

- Written

- Oral Interview

LO3.	ANALYZE RESULTS

# ASSESSMENT CRITERIA:

  - Results of application based on expected and required specifications and outcome is reviewed.

  - Appropriate action in case of error is applied.

CONTENTS:

- Four fundamental operations

- Steps in solving a problem

- Standard formulas

- Conversion

- Measurement

CONDITION:

The students/learners must be provided with the following:

- Manuals

- Hand-outs

- Problem set

- Conversion table

- Table of formulas

- Measuring tools

METHODOLOGIES:

- Lecturette

- Self-paced instruction

- Group discussion

- Research study

ASSESSMENT METHODS:

- Written

- Oral

##### UNIT OF COMPETENCY	:	USE RELEVANT TECHNOLOGIES

##### MODULE TITLE	:	USING RELEVANT TECHNOLOGIES

MODULE DESCRIPTOR	:	This module covers the knowledge, skills and attitudes required in selecting, sourcing and applying appropriate and affordable technologies in the workplace.

NOMINAL DURATION	:	3 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of the module, the learner/students must be able to:

LO1.	Identify appropriate technology

LO2.	Apply relevant technology.

LO3. 	Maintain/ enhance relevant technology

LO1.	IDENTIFY appropriate technology

ASSESSMENT CRITERIA:

- Appropriate technologies are studied based on work requirements.

- Appropriate technologies are identified and selected based on work requirements.

CONTENTS:

- Machineries/ equipment and their application

- Software/ programs

CONDITION:

The students/learners must be provided with the following:

- Manuals

- Hand-outs

- Multimedia

- Video tape

- Brochures

- CD’s

- Internet access

- Computer

METHODOLOGIES:

- Lecturette

- Self-paced instruction

- Group discussion

- Film showing

ASSESSMENT METHODS:

- Written

- Interview

LO2.	Apply relevant technology

# ASSESSMENT CRITERIA:

1.	Relevant technology is used in carrying out function based on work requirements.

2.	Applicable software and hardware is used as per job requirement.

3. 	Management concepts are observed as per established industry practices.

CONTENTS:

- Office technology

- Industrial technology

- System technology

- Information technology

- Training technology

- Different software/ hardware

- 5S (proper house keeping)

CONDITION:

The students/learners must be provided with the following:

- Manuals

- Hand-outs

- Multimedia

- Video tape

- Brochures

- CD’s

- Internet access

- Computer

METHODOLOGIES:

- Lecturette

- Self-paced instruction

- Group discussion

- Film showing

ASSESSMENT METHODS:

- Written

- Interview

LO3.	MAINTAIN / ENHANCE RELEVANT TECHNOLOGY

# ASSESSMENT CRITERIA:

- Maintenance of technology is applied in accordance with the industry standard operating procedure, manufacturer’s operating guidelines and occupational health and safety procedure

- Updating of technology is maintained through continuing education or training in accordance with job requirement.

- Appropriate action for technology failure/defect is immediately reported to the concerned/ responsible person or section.

CONTENTS:

- Corrective and preventive maintenance

- Upgrading of technology

- Communication Skills

- Organizational set – up / work flow

CONDITION:

The students/learners must be provided with the following:

- Manuals

- Hand-outs

- Multimedia

- Video tape

- Brochures

- CD’s

- Internet access

- Computer

METHODOLOGIES:

- Lecturette

- Self-paced instruction

- Group discussion

- Film showing

ASSESSMENT METHODS:

- Written

- Interview

MODULES OF INSTRUCTION

COMMON COMPETENCIES

AUTOMOTIVE SERVICING NC III

UNIT OF COMPETENCY	:	APPLY APPROPRIATE SEALANT/ADHESIVE

MODULE TITLE	:	APPLYING APPROPRIATE SEALANT/ADHESIVE

MODULE DESCRIPTOR	:	This module covers the selection and application of sealant/adhesives particularly to identify appropriate sealant and adhesives, prepare surface for sealant and adhesive, apply sealant/adhesives evenly, and store unused/dispose used sealant/adhesive properly.

NOMINAL DURATION	:	3 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module, the trainee/student must be able to:

LO1.	Identify appropriate sealant/adhesive

LO2. 	Prepare surface for sealant/adhesive application

LO3.	Store unused and dispose used sealant/adhesive.

LO1.	IDENTIFY APPROPRIATE SEALANT/ADHESIVE

ASSESSMENT CRITERIA:

- Appropriate sealant and adhesives are identified.

- Sealant/adhesive is selected in line with job requirements and manufacturer’s specification.

- Sealant/adhesive checking is performed to ensure that product is fit for use.

- Work safety procedures are applied.

CONTENTS:

- Use of sealant and adhesive

- Types and classification of sealant and adhesives

CONDITION:

Students/trainees must be provided with the following:

- Equipment/tools/instruments

- Vehicle

- Car lift/bay

- Putty knife

- Gloves

- Apron

- Mask

- Scraper

- Supplies/materials

- Gasket

- Ribbon sealer

- Different types of sealant/adhesives

- Silicon

- Instructional materials

- Reference books

- Learning materials

- Modules

METHODOLOGIES:

- Lecture/demonstration

- Dualized training

- Distance learning

ASSESSMENT METHODS:

- Written/oral examinations

- Direct observation

- Project method

- Interview

LO2.	PREPARE SURFACE FOR SEALANT/ADHESIVE APPLICATION

ASSESSMENT CRITERIA:

- Types of sealant and adhesives are identified according to surface.

- Surface materials are identified as per construction.

- Surface is cleaned and free of moisture, dust and other foreign matters to ensure maximum adhesion or seal.

- Safety is observed while performing job.

- Sealant/adhesive is applied evenly on the surface in line with manufacturer’s specification.

- Excess sealant/adhesive is removed by sanding or by scrapping.

- Tools and equipment used to apply sealant/adhesive are appropriate to job requirements.

- Safety is observed and PPE are worn in accordance with industry SOP.

- Hazards associated with the use of sealant and adhesives are identified.

CONTENTS:

- Types of surface material and appropriate sealant/adhesive to be used.

- Techniques and procedures in preparing surfaces for sealant/adhesive.

- Use of sealant and adhesive

- Types and classification of sealant and adhesives

- Tools and materials to be used in applying sealant/adhesive

- Techniques in applying sealant/adhesive

- Safety requirements in applying sealant/adhesive

CONDITION:

Students/trainees must be provided with the following:

- Equipment/tools/instruments

- Vehicle

- Putty knife

- Gloves

- Apron

- Mask

- Scraper

- Sander

- Supplies/materials

- Paper

- Pencil

- Solvent

- Sand paper

- Instructional materials

- Reference books

- Learning materials

- Modules

- CDs, VHS tapes, transparencies

METHODOLOGIES:

- Lecture/demonstration

- Dualized training

- Distance learning

ASSESSMENT METHODS:

- Written/oral examination

- Direct observation

- Project method

- Interview

LO3.	STORE UNUSED AND DISPOSE USED SEALANT/ADHESIVE

ASSESSMENT CRITERIA:

- Sealant/adhesive is stored as per prescribed procedure.

- Wastes are disposed as per workshop SOP.

- Safe handling of sealant/adhesive is observed.

CONTENTS:

- Sealant/adhesive storing procedures.

- Waste disposal standard procedures

- Hazards associated with improper waste disposal

CONDITION:

Students/trainees must be provided with the following:

- Equipment/tools/instruments

- Vehicle

- Car lift/bay

- Putty knife

- Gloves

- Apron

- Mask

- Scraper

- Sander

- Supplies/materials

- Paper

- Pencil

- Solvent

- Sand paper

- Instructional materials

- Reference books

- Learning materials

- Modules

- CDs, VHS tapes, transparencies.

METHODOLOGIES:

- Lecture/demonstration

- Dualized training

- Distance learning

ASSESSMENT METHODS:

- Written/oral examination

- Direct observation

- Project method

- Interview

UNIT OF COMPETENCY	:	MOVE AND POSITION VEHICLE

MODULE TITLE	:	MOVING AND POSITIONING VEHICLE

MODULE DESCRIPTOR	:	This module covers knowledge skills and attitudes needed to move and position vehicle. This requires driving, checking vehicle inside and outside of the work area.

NOMINAL DURATION	:	3 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of the module, the trainee/students should be able to:

LO1.	Prepare vehicle for driving

LO2.	Move and position vehicle

LO3.	Check the vehicle

LO1.	PREPARE THE VEHICLE FOR DRIVING

ASSESSMENT CRITERIA:

1.	Correct check-up procedures of engine system is performed.

2.	Tire inflation pressures are checked according to manufacturer’s specification.

- Braking system and fluid level are properly checked.

- Engine belt tension and other necessary units are checked in accordance with manual.

- Lighting systems are checked according to established procedures.

CONTENTS:

- Safety in driving.

- Engine system check up procedures.

- Tire inflation pressure specification.

- Belt tension checking procedures.

- Driver’s code and conduct.

CONDITIONS:

Students/trainees must be provided with the following:

- Equipment/tools/instrument

- Good running condition vehicle

- Car lift/bay

- Belt tension gauge

- Supplies/materials

- Paper

- Pencil

- Brake fluid/engine oil/coolant

- Instructional materials

- Reference books

- Learning guides

- Modules

- CDs, VHS tapes, transparencies

METHODOLOGIES:

- Lecture

- Demonstration

- Dualized training

- Distance learning

- Film viewing

ASSESSMENT METHODS:

- Written and/or oral examination

- Direct observation

- Project-based

- Interview

LO2.	MOVE AND POSITION VEHICLE

ASSESSMENT CRITERIA:

- Vehicle to be moved or repositioned is identified or selected as per job requirements.

- Vehicle is driven safely to the designated location according to job specification.

- Vehicle is parked properly following safety procedures and techniques.

CONTENTS:

- Starting an engine

- Shifting gears.

- Steering vehicle

- Brake application

- Driving skills

CONDITIONS:

Students/trainees must be provided with the following:

- Equipment/tools/instrument

- Functional, good running condition light vehicle

- Car lift/bay

- Belt tension gauge

- Supplies/materials

- Paper

- Pencil

- Instructional materials

- Reference books

- Learning guides

- Modules

- CD’s, VHS tapes, transparencies

METHODOLOGIES:

- Lecture/demonstration

- Dualized training

- Distance learning

ASSESSMENT METHODS:

- Written/oral examination

- Direct observation

- Project-based

- Interview

LO3.	CHECK THE VEHICLE

ASSESSMENT CRITERIA:

- Vehicle position is checked as per requirement.

- Check up procedures is performed upon parking.

- Vehicle is checked for external damage.

CONTENTS:

- Oil level

- Brake fluid level

- Battery electrolytes

- Tire pressure

- Clutch fluid

- Position of driving gear

- Lighting and warning devices

CONDITIONS:

Student/trainees must be provided with the following:

- Equipment/tools/instrument

- Functional, good running condition light vehicle

- Car lift/ bay

- Supplies/materials

- Paper

- Pencil

- Instructional materials

- Reference books

- Learning guides

- Modules

- CD’s, VHS tapes, transparencies

METHODOLOGIES:

- Lecture and or demonstration

- Dualized training

- Distance learning

- Film viewing

ASSESSMENT METHODS:

- Written and/or oral examination

- Direct observation

- Project-based

- Interview

UNIT OF COMPETENCY	:	Perform Mensuration and Calculation

MODULE TITLE	:	Performing mensuration and calculation

MODULE DESCRIPTOR	:	This module covers calculations and mathematical problems related to automotive works including tolerances and measurement, including conversion of two systems of measurements.

NOMINAL DURATION	:	4 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of the module, the trainees/students should be able to:

LO1.	Select measuring instrument and	carry out measurement and calculation.

LO2.	Maintain measuring instruments.

LO1.	SELECT MEASURING INSTRUMENT AND CARRY OUT MEASUREMENT AND CALCULATION

ASSESSMENT CRITERIA:

- Objects or components to be measured are identified.

- Correct specifications are obtained from relevant source.

- Appropriate measuring instrument is selected as per job requirement.

- Measuring tools are selected in line with job requirements.

- Accurate measurements are obtained.

- Calculations needed to complete work/task are performed using the four fundamental operation of mathematics.

- Numerical computation checked is correct and accurate.

CONTENTS:

- Types of measuring instruments and applications.

- Techniques in measuring parts/components.

- Methods of calculating/solving mathematical problems.

- Conversion of units of measurement from English-metric and vise versa.

- Techniques in determining tolerance/allowance of parts/components

- Calibrating and using testing instruments.

- Solving problems using formulas.

- Finding areas of different geometrical figures.

CONDITIONS:

- Equipment/tools/instrument

- Precise measuring instruments

- Calculator

- Calipers-vernier, micrometer, bore gauge, feeler gauge

- Steel rule, push rule

- Supplies/materials

- Pencil

- Paper

- Models of engine parts/components

- Instructional materials

- Reference books

- Learning guides

- Modules

METHODOLOGIES:

- Lecture

- Demonstration

- Exercises

ASSESSMENT METHODS:

- Written and oral examination

- Direct observation

LO2.	MAINTAIN MEASURING INSTRUMENT

ASSESSMENT CRITERIA:

- Measuring instruments are cleaned before and after using.

- Measuring instruments are kept in safe dry place.

- Proper techniques in using precise instrument are according to manufacturer’s standards.

- Proper handlings of measuring instruments are observed in accordance with industry standards.

CONTENTS:

- Handling and caring of measuring instruments.

- Calibrating measuring instrument.

- Safe handling and procedures in using measuring instruments.

- Storing of measuring instruments.

CONDITIONS:

Students/trainees must be provided with the following:

- Equipment/tools/instrument

- Computer

- VHS player

- OHP

- Vernier calipers, micrometer, bore gauge, feeler gauge

- Steel rule, push rule

- Supplies/materials

- Paper

- Pencil

- Engine parts

- Instructional materials

- Reference books

- Learning guides

- Modules

- CD’s VHS tapes, transparencies

METHODOLOGIES:

- Lecture

- Demonstration

- Simulation

ASSESSMENT METHODS:

- Written examination

- Direct observation

- Oral examination

UNIT OF COMPETENCY	:	READ, INTERPRET AND APPLY SPECIFICATION AND MANUAL

MODULE TITLE	:	READING, INTERPRETING AND APPLYING SPECIFICATION AND MANUAL

MODULE DESCRIPTOR	:	This module deals with identifying, interpreting and applying service manuals, maintenance procedures manuals, repair manuals and periodical maintenance manual.

NOMINAL DURATION	:    4 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of the module, the trainees/ students should be able to:

LO1.	Identify/access manuals and interpret data and specification.

LO2.	Apply information accessed in manual.

LO3.	Store manual

LO1.	IDENTIFY/ACCESS MANUALS AND INTERPRET DATA AND SPECIFICATIONS

ASSESSMENT CRITERIA:

- Appropriate manuals are identified and accessed as per job requirements.

- Information and procedures in the manual are interpreted in accordance with industry practices.

- Relevant sections, chapter of manuals/specifications are located in relation to the work to be performed.

CONTENTS:

- Identifying appropriate manuals.

- Procedure/ techniques in interpreting data and specifications.

- Knowledge and techniques in accessing data and specifications as per job requirements.

- Types of manuals used in automotive industry.

- Identification of symbols used in the manuals.

- Identification of units of measurements.

CONDITIONS:

Students/trainees must be provided with the following:

- Supplies and materials:

- manufacturing specifications

- repair manual

- maintenance manual

- periodic maintenance manual

- Instructional materials

- reference books

- professional emergency

- learning guides

- modules

- CD’s, VHS tapes, transparencies

METHODOLOGIES:

- Lecture/demonstration

- Dualized training

- Distance learning

ASSESSMENT METHODS:

- Written/oral examinations

- Direct observation

- Project method

- Interview

LO2.	APPLY INFORMATION ACCESSED IN THE MANUAL

ASSESSMENT CRITERIA:

- Data and specification are interpreted according to job requirement.

- Work steps are correctly identified in accordance with manufacturer’s specifications.

- Manual data is applied according to the given task.

- All correct sequence and adjustment are interpreted in accordance with information contained on the manual or specification.

CONTENTS:

- Interpreting appropriate data and specifications.

- Applying data and specification accessed from the manuals as required in the given task.

CONDITIONS:

The students/trainees must be provided with the following:

- Supplies and materials

- manufacturing specifications

- repair manual

- maintenance manual

- periodic maintenance manual

- Instructional  materials

- reference books

- learning guides

- modules

- CD’s, VHS tapes, transparencies

METHODOLOGIES:

- Lecture/demonstration

- Dualized training

- Distance learning

ASSESSMENT METHODS:

- Written/oral examinations

- Direct observation

- Project method

- Interview

LO3.	STORE MANUAL

ASSESSMENT CRITERIA:

- Manuals are stored appropriately to insure prevention of damage.

- Manuals are stored properly for easy accessed and ready for updating of information required in the given task.

CONTENTS:

- Techniques in storing manuals.

- Procedures in maintaining manuals.

CONDITIONS:

Students/trainees must be provided with the following:

- Supplies/materials

- manufacturing specifications

- repair manual

- maintenance manual

- periodic maintenance manual

- Instructional materials

- reference books

- learning guides

- modules

- CD’s, VHS tapes, transparencies

METHODOLOGIES:

- Lecture/demonstration

- Dualized training

- Distance learning

ASSESSMENT METHODS:

- Written/oral examinations

- Direct observation

- Project method

- Interview

UNIT OF COMPETENCY	:	USE AND APPLY LUBRICANT/COOLANT

MODULE TITLE	:	USING AND APPLYING LUBRICANT/COOLANT

MODULE DESCRIPTOR	:	This module covers identification of different lubricant, coolants, and its uses and application procedures.

NOMINAL DURATION	:	3 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module, the trainee/student must be able to:

LO1.	Identify the type of lubricant/coolant

LO2.	Use and apply lubricants/coolant

LO1.	IDENTIFY THE TYPE OF LUBRICANT/COOLANT

ASSESSMENT CRITERIA:

- Correct information on lubrication schedule is accessed and interpreted from appropriate manufacturer’s specifications manuals.

- Type and quantity of lubricants/coolant is identified as per job requirements.

CONTENTS:

- Lubrication schedules

- Uses of coolants

- Properties of lubricant and coolant

- Types of lubricant and application

- Hazards associated with lubricants.

CONDITIONS:

Students/trainees must be provided with the following:

- Supplies/materials

- Engine oil

- Gear oil

- Coolant/water

- Instructional materials

- Reference books

- Periodic maintenance

- Lubricating schedule

- Learning guides

- Modules

- CDs, VHS, tapes, transparencies

METHODOLOGIES:

- Lecture/demonstration

- Dualized training

- Distance learning

ASSESSMENT METHODS:

- Written/oral examination

- Direct observation

- Project method

- Interview

LO2.	USE AND APPLY LUBRICANTS/COOLANT

ASSESSMENT CRITERIA:

- Correct procedure for change of lubricant is identified following manufacturer’s specification or manual.

- Correct tools and equipment are selected and used in line with job requirements.

- Existing lubricants are removed and replaced with specified types and quantity of new materials in line with manufacturer’s specification.

- Safe procedure and use of PPE is observed when removing or replacing lubricant.

- Used lubricants are disposed in accordance with environmental guidelines.

- Work is checked in line with company SOP.

CONTENTS:

- Lubrication schedule

- Hazards associated with lubricant

- Lubrication procedure

- Techniques in handling lubricants

- Personal safety procedures

- Use and care of tools and equipment

CONDITIONS:

Students/trainees must be provided with the following:

- Equipment/tools/instruments

- Vehicle

- Engine

- Set of wrenches

- Oiler

- Apron

- Gloves

- Safety shoes

- Goggles

- Grease gun

- Supplies/materials

- Engine oil

- Grease

- Gear oil

- Power steering fluid

- Radiator coolant

- Instructional materials

- Reference books

- Periodic maintenance manual

- Service manual

- Learning guides

- Modules

- CDs, VHS, tapes, transparencies

METHODOLOGIES:

- Lecture/demonstration

- Dualized training

- Distance learning

ASSESSMENT METHODS:

- Written/oral examination

- Direct observation

- Project method

- Interview

UNIT OF COMPETENCY	:	Perform Shop Maintenance

MODULE TITLE	:	PERFORMING SHOP MAINTENANCE

MODULE DESCRIPTOR	:	This module covers inspecting, cleaning, arranging and storing tools and shop equipment, including disposal of waste and used lubricant and reporting of damaged tools and equipment in the shop.

NOMINAL DURATION	:	3 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module, the trainee/student must be able to:

LO1.	Inspect/clean tools and shop equipment

LO2.	Store/arrange tools and shop equipment

LO3.	Dispose waste/used lubricant

LO4.	Report damaged tools/equipment

LO1.	INSPECT/CLEAN TOOLS AND SHOP EQUIPMENT

ASSESSMENT CRITERIA:

- Tools, equipment and the work area is inspected and cleaned, free from dust, grease and other substances.

- Cleaning solvent is used as per workshop cleaning requirements is observed.

- Work area is checked and cleaned.

- Wet surface or spot in the work area is kept dry.

CONTENTS:

- Workshop policies.

- Types and usage of cleaning chemicals.

- Safe handling of equipment and tools.

- Service procedures.

- Equipment maintenance standards.

CONDITIONS:

Students/trainees must be provided with the following:

- Open workshop/garage

- Well ventilated enclosed work and office area

- Wash area and mess hall

- Cleaning chemicals

- Inventory of equipment and tools

- Rugs

- Containers for used oils

- Vehicle

- Oiler

METHODOLOGIES:

- Lecture

- Demonstration

- Self-paced instruction

- Dual training

ASSESSMENT METHODS:

- Direct observation

- Practical

- Interview

- Written examination

LO2.	STORE/ARRANGE TOOLS AND SHOP EQUIPMENT

ASSESSMENT CRITERIA:

1.	Tools and equipment are arranged and stored in their respective shelves/ location.

2.	Corresponding labels are posted and visible.

3.	Tools are secured and logged in the record book.

CONTENTS:

- Storage and disposal of hazardous/flammable tools/materials.

- Personal safety procedures.

- Relevant technical information

- Labeling procedures

- Principles of total quality management (TQM) and 5S

CONDITIONS:

Students/trainees must be provided with the following:

- Tool cabinet

- Appropriate tools and equipment

- Tool room

- Fire extinguishers

- Tool kit

- Log/record  book

- Labeling materials

METHODOLOGIES:

- Interactive lecture

- Dual Training

- Self-paced instruction

- Simulation

ASSESSMENT METHODS:

- Practical

- Direct observation

- Interview

- Written examination

LO3.	DISPOSE WASTE/USED LUBRICANT

ASSESSMENT CRITERIA:

- Waste and used lubricants are disposed in accordance with the standard operational procedures and environmental regulations.

- Containers for waste and used lubricants are properly labeled.

- Personal safety in disposing waste and used lubricants is evident.

CONTENTS:

- Effects of automotive wastes to men and its environment.

- Waste management and disposal

- Cleaning chemicals for grease and lubricants

- Labeling procedures and technique

CONDITIONS:

Students/trainees must be provided with the following:

- Waste disposal area

- Oiler

- Handouts for waste management and disposal

- Cleaning chemicals for grease and lubricants

- Containers for waste and used lubricants

METHODOLOGIES:

- Interactive lecture

- Dual training

- Self-paced instruction

- Simulation

- Demonstration

ASSESSMENT METHODS:

- Direct observation

- Simulation

- Written examination

- Interview

LO4.	REPORT DAMAGED TOOLS/EQUIPMENT

ASSESSMENT CRITERIA:

- Complete inventory of tools and equipment is maintained.

- Damaged tools/equipment is identified with repair recommendation.

- Reports prepared on damaged tools/equipment have no error/discrepancy.

CONTENTS:

- Conducting inventory and preparing records

- Maintenance and safe handling of tools and equipment

- Maintenance and updating of records and reports

CONDITIONS:

- Records and inventory of tools and equipment

- Record books

- Checklist

- Waste disposal reports

- Racks for tools

METHODOLOGIES:

- Interactive lecture

- Dual Training

- Self-paced instruction

- Simulation

ASSESSMENT METHODS:

- Interview

- Written examination

- Demonstration

- Direct observation

MODULES OF INSTRUCTION

CORE COMPETENCIES

AUTOMOTIVE SERVICING NC III

UNIT OF COMPETENCY	:	Test and Repair Electrical Security System/Components

MODULE TITLE 	:	TESTING & REPAIRING ELECTRICAL SECURITY SYSTEM COMPONENTS

MODULE DESCRIPTOR	:	This module covers installation testing & repair of Auto. Electrical Security system components

NOMINAL DURATION	: 	40 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module, the trainee/student must be able to:

LO1. 	Install electrical security system/components

LO2.	Test and repair anti-theft system and components

## LO1.	INSTALL Electrical SECURITY System/Components

ASSESSMENT CRITERIA:

- Correct information is accessed and interpreted from manufacturer specs.

- Appropriate fitting/materials and tools are selected.

- Electrical security system/components is installed and wired according to Industry procedures.

CONTENTS:

- Use of tools & equipments

- Safety rules in Auto Electrical works

- Interpret wiring diagram

- Interpret correct information

- Installing electrical security system

CONDITIONS:

Students/trainees must be provided with the following:

- Fender cover

- Service manual

- Checklist

- Testing instrument

- Load tester

- Multi tester

- Vehicle

- Box & open wrench

- Pliers

- Screw drivers

METHODOLOGIES:

- Traditional/lecture type

- Dual training

- Self-paced instruction

- Community based

ASSESSMENT METHODS:

- Interview

- Written

- Practical

- Direct observation

# LO2.	Test and Repair Anti-ThEft System AND COMPONENTS

ASSESSMENT CRITERIA:

- Correct Information is accessed and interpreted.

- Test is carried out and fault is determined using appropriate tools & fitting.

- Preferred repair action is recommended.

- Electrical security system defective parts are replaced/repaired.

CONTENTS:

- Repair/clean & replace connectors

- Observe polarity connection

- Interpret correct information

- Methods of finding faults

- Procedure in replacing defective parts of electrical security system

CONDITIONS:

Students/trainees must be provided with the following:

- VOM Tester

- Multi tester

- Electrical Tapes

- Tools required

- Box & open wrenches

- Pliers

- Screw driver

- Fender cover

- Vehicle

METHODOLOGIES:

- Traditional/lecture type

- Dual training

- Self-paced instruction

- Community based

- Distance learning

ASSESSMENT METHODS:

- Interview

- Written

- Practical

- Direct observation

UNIT OF COMPETENCY	:	SERVICE ELECTRONIC ENGINE MANAGEMENT SYSTEM

MODULE TITLE 	:	SERVICING ELECTRONIC ENGINE MANAGEMENT SYSTEM

MODULE DESCRIPTION	:	This module identifies the competence required to service/repair electronic fuel injection and electronic engine management systems.

SUGGESTED DURATION	:	40 hours

PREREQUISITE	:	Service ignition system, Service engine mechanical system

- SUMMARY OF LEARNING OUTCOMES:

At the completion of the module the trainees/students should be able to:

LO1. 	Check electronic engine management system and/or associated components

LO2. 	Service electronic engine management associated parts

LO1.	Check electronic engine management system and/or associated components

- ASSESSMENT CRITERIA:

- Air induction and air flow meter is identified.

- Map sensor is tested.

- Throttle position sensor is identified.

- Filter is cleaned.

- Fuel injection cold start injection is inspected.

- Pressure regulators are measured.

- Temperature sensor is inspected.

- CONTENTS:

- Major components of engine management system.

- Procedure in testing and determining components, defects and its solution.

- Engine tune-up procedure.

- Safety in handling electronic cars.

- CONDITION:

Students/trainees must be provided with the following:

- Jetronic or TCCS equipment car

- Simulated work place

- Special tools and tester

- Tape, CDs, transparency

- Safety gadgets

- METHODOLOGIES:

- Lecture/demonstrations

- Self-paced instruction

- ASSESSMENT METHODS:

- Knowledge

- Practical

- Interview

- Direct observation

LO2.	Service electronic engine management associated parts

- ASSESSMENT CRITERIA:

- Electronic servicing tools and instruments are used.

- Sensors are tested.

- ECU is inspected.

- Actuator is inspected and tested.

- Circuit diagrams are traced.

- Defective parts are replaced and loose circuits are repaired.

- CONTENTS:

- Jetronic principles.

- Procedure in testing ECU, sensor and actuator.

- Interpreting specification manuals and diagram.

- Safety in handling electronic/electricity components.

- Exhaust gas analysis.

- Clean air act.

- Repairing electronic circuits and replacing defective parts

- CONDITIONS:

Students/trainees must be provided with the following:

- Simulated workplace

- Jetronic/EFI equipment car

- Engine analyzer

- ECU diagnostic connector

- Manuals

- Special tools

- Fuel (gasoline unleaded)

- METHODOLOGIES:

- Lecture/demonstrations

- Self-paced instruction

- ASSESSMENT METHODS:

- Knowledge

- Practical

- Interview

- Direct observation

UNIT OF COMPETENCY	:	OVERHAUL ENGINE AND ASSOCIATED COMPONENTS

MODULE TITLE	:	INSPECTING AND ANALYZING ENGINE FAILURE

MODULE DESCRIPTOR 	:	This module covers the knowledge, skills and attitudes required to overhaul engines and associated components of a removed engine. The task involves complete dismantling and rebuilding of light/heavy vehicles, plant, motorcycles and marine on 2 and 4 stroke spark ignition and 2 and 4 stroke compression ignition engines.

NOMINAL DURATION	:	80 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module, the trainee/student must be able to:

LO1.	Analyze engine failure

LO2.	Pull down the engine

LO3.	Disassemble engine parts and components

LO4.	Inspect engine parts and components

LO5.	Perform fitting of engine parts

LO6.	Assemble engine parts and components

LO7.	Mount and test run engine

###### LO1.	ANALYZE ENGINE FAILURE

ASSESSMENT CRITERIA:

- Battery is checked and used to test the condition of the engine.

- Cylinder compression is checked

- Valve timing is inspected

- Oil pressure is checked

- Cooling system (radiator or leak) is inspected

- Valve clearance is inspected/adjusted

- High tension cord is disconnected to the ignition coil during engine cranking

CONTENTS:

- Principles of 2/4 stroke cycle engine

- Engine internal and external parts.

- Difference between spark ignition and compression ignition engine

- Compression testing

- Valve clearance and timing

- Oil pressure and gauge reading

- Cooling system parts and functions

- Ignition system parts and functions

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle with 2/4 stroke cycle engine

- Manufacturer’s specification manual

- Maintenance procedure manual

- Periodic maintenance data

- Service manual

- Compression tester

- Feeler gauge

- Multitester

METHODOLOGIES:

- Traditional/lecture/demonstration

- Dual training

- Self-paced instruction

ASSESSMENT METHODS:

- Observation with questioning

- Demonstration with questioning

###### LO2.	PULL DOWN THE ENGINE

ASSESSMENT CRITERIA:

- Vehicle is parked on designated area and wheel choke is applied.

- Disconnected wire harness is accurately marked.

- Battery is disconnected before proceeding to remove other connections.

- Engine is securely fitted or bolted to the engine stand.

CONTENTS:

- Procedure in pulling down the engine from the vehicle.

- Using lifting/jacking equipment

- Procedure in disconnecting engine wirings and accessories

- Procedure in mounting the engine to the engine stand

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle with 2/4 stroke cycle engine

- Safety shoes

- Apron

- Goggles

- Hand gloves

- Hand tools

- Lifting and jacking equipment

- Engine stand

METHODOLOGIES:

- Traditional/lecture/demonstration

- Dual training

- Self-paced instruction

ASSESSMENT METHODS:

- Observation with questioning

- Demonstration with questioning

###### LO3.	DISASSEMBLE ENGINE PARTS AND COMPONENTS

ASSESSMENT CRITERIA:

- Engine parts are checked for wear, damage and dirt

- Head bolts are removed following sequential order.

- Oil pan is removed.

- Crankshaft is removed in sequential order as specified in the manufacturer’s manual

CONTENTS:

- Parts and components of the engine

- Procedure in disassembling engine parts and components

- Engine parts/components handling techniques

- Cleaning the engine

CONDITIONS:

Students/trainees must be provided with the following:

- Engine mock-up 2/4 stroke cycle.

- Required hand tools

- Specialist tools

- Cleaning solvent

- Rags

METHODOLOGIES:

- Traditional/lecture/demonstration

- Dual training

- Self-paced instruction

ASSESSMENT METHODS:

- Observation with questioning

- Demonstration with questioning

###### LO4.	INSPECT ENGINE PARTS AND COMPONENTS

ASSESSMENT CRITERIA:

- Timing belt is not bended, twisted or turned inside out

- Belt is safeguarded not to get in contact with oil or other liquid chemical

- Cylinder warpage is referred to manufacturer’s manual

- Crankshaft maximum run-out is  0.06m

- Inspection of the following should conform to manufacturer’s manual

- Main journal diameter

- Valve stem or valve bushing clearance

- Valve face angle is 45.5º

- Camshaft bore diameter oil clearance

- Piston ring groove clearance

- Piston ring end gap

- Piston to cylinder bore clearance

- Cylinder bore out of round & taperness

- Cam lobe diameter

- Cam lift

- Crankshaft main journal/connecting rod journal out-of-round and taperness

- Thrust clearance

- Connecting rod alignment

- Valve spring tension

CONTENTS:

- Engine overhaul procedures

- Measuring and testing procedures

- Relevant technical information

- Equipment safety requirements

- Engine component safety requirements

- Relevant enterprise policies

CONDITIONS:

Students/trainees must be provided with the following:

- Engine mock-up 2/4 stroke cycle.

- Required hand tools

- Specialist tools

- Measuring equipment

METHODOLOGIES:

- Traditional/lecture/demonstration

- Dual training

- Self-paced instruction

ASSESSMENT METHODS:

- Observation with questioning

- Demonstration with questioning

###### LO5.	PERFORM FITTING OF ENGINE PARTS

ASSESSMENT CRITERIA:

- Fitting/setting of valve to valve seat.

- Engine valves are fitted to the engine.

- Camshaft is fitted to cylinder head.

- Plastic gage is used to measure engine warpage

- Bolts and nuts are tightened in sequence with correct torque.

- Camshaft alignment and timing marks are matched.

- Replacement pistons are used in accordance with the manufacturer’s specifications.

CONTENTS:

- Procedures in replacing damaged engine parts/components

- Measuring and testing procedures

- Relevant technical information

- Equipment safety requirements

- Engine component safety requirements

- Relevant enterprise policies

CONDITIONS:

Students/trainees must be provided with the following:

- Engine mock-up 2/4 stroke cycle.

- Required hand tools

- Specialist tools

- Measuring equipment

- Tensioning equipment

- Engine oils

- Moving part lubricants

- Replacement parts

- Gaskets and sealant

METHODOLOGIES:

- Traditional/lecture/demonstration

- Dual training

- Self-paced instruction

ASSESSMENT METHODS:

- Observation with questioning

- Demonstration with questioning

###### LO6.	ASSEMBLE ENGINE PARTS AND COMPONENTS

ASSESSMENT CRITERIA:

- Bolts are tightened in sequence.

- Valve timing marks are aligned.

- Measurement of timing belt deflection is 5-6 mm at 2 kg force.

- Valve clearance is adjusted.

CONTENTS:

- Procedure in assembling the engine.

- Relevant technical information

- Equipment safety requirements

- Engine component safety requirements

- Relevant enterprise policies

CONDITIONS:

Students/trainees must be provided with the following:

- Engine mock-up 2/4 stroke cycle.

- Required hand tools

- Specialist tools

- Measuring equipment

- Tensioning equipment

- Engine oils

- Moving part lubricants

- Replacement parts

- Gaskets and sealant

METHODOLOGIES:

- Traditional/lecture/demonstration

- Dual training

- Self-paced instruction

ASSESSMENT METHODS:

- Observation with questioning

- Demonstration with questioning

###### LO7.	MOUNT AND TEST RUN ENGINE

ASSESSMENT CRITERIA:

- Engine is positioned to the vehicle supported by lifting equipment.

- Accurate and correct connection of engine wiring harness, fuel lines and battery.

- All mounting bolts are tightened as per specified torque.

CONTENTS:

- Procedure in mounting the engine to the vehicle

- Using lifting equipment

- Procedure in reconnecting engine wirings and accessories to the vehicle

- Nuts and bolts tightening sequence and specified torque

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle with 2/4 stroke cycle engine

- Safety shoes

- Apron

- Goggles

- Hand gloves

- Hand tools

- Lifting and jacking equipment

METHODOLOGIES:

- Traditional/lecture/demonstration

- Dual training

- Self-paced instruction

ASSESSMENT METHODS:

- Observation with questioning

- Demonstration with questioning

UNIT OF COMPETENCY	:	SERVICE AUTOMATIC TRANSMISSION

MODULE TITLE	:	INSPECTING AND DIAGNOSING AUTOMATIC TRANSMISSION

MODULE DESCRIPTION	:	This module deals with overhauling automatic transmission (A/T).  It also includes testing of (A/T) before and during road test.

SUGGESTED DURATION	:	24 hours

- SUMMARY OF LEARNING OUTCOMES:

At the completion of the module the trainees/students should be able to:

LO1. 	Explain the principles and components of automatic transmission.

LO2. 	Diagnosing automatic transmission

LO1.	Explain the principles and components of automatic transmission

ASSESSMENT CRITERIA:

- Components are identified according to function.

- Transmission fluid is drained completely.

- Power flow is explained.

- Operation of automatic transmission is explained.

CONTENTS:

- Engine condition

- Fluid level

- Fluid condition

- Control Linkage

- Vacuum modulator

- Pressure testing

- Air checking

- Manufacturer’s specification manual

- Maintenance procedure manual

- Periodic maintenance manual

- Service and repair manual

- Parts checklist

CONDITIONS:

Students/trainees must be provided with the following:

- Automatic transmission

- Hydraulic lifter/mechanical lifter

- Transmission jack

- Socket wrench

- Snap ring expander

- Snap ring compressor

- Micrometer

- Torch wrench

- Allen wrench

- Combination Wrench

- Basic hand tools

- A/T pressure gauge

- Torque wrench

- Dial indicator with stand

- Air compressor

- Hand rubber mallet

- Ball peen hammer

- METHODOLOGIES:

- Lecture

- Self-paced instruction

LO2.	Diagnosing automatic transmission

ASSESSMENT CRITERIA:

- Automatic transmission is tested for noise.

- Shifting conditions is checked.

- Leak of fluid is inspected.

- Contamination of gear oil is discovered.

- Fluid pressure is tested.

CONTENTS:

- Develop standard test route

- Test route include level section, hills section and quiet section

A/T must operate in each range to check shifting, check upshift and down shift speeds and check for evidence of slippage refer to band chart.

- Isolate A/T noise from various chassis and body components noises such as air-conditioning alternator, power steering and fan by removal of drive belts of accessories and drive vehicle a short distance.

CONDITIONS:

Students/trainees must be provided with the following:

- Hydraulic lifter/mechanical lifter

- Support stand

- Transmission Jack

- Socket Wrench

- Snap ring expander

- Snap ring compressor

- Micrometer

- Torch Wrench

- Allen Wrench

- Combination Wrench

- Basic handtools

- A/T pressure gauge

- Torque Wrench

- Dial indicator with stand

- Air compressor

- Hard Rubber mallet

- Ball peen hammer

METHODOLOGIES:

- Lecture

- Self-paced instruction

ASSESSMENT METHODS:

- Practical and written test

- Interview

- Direct observation

UNIT OF COMPETENCY	:	SERVICE AUTOMATIC TRANSMISSION

MODULE TITLE	:	PULLING OUT DISASSEMBLING AND CHECKING AUTOMATIC TRANSMISSION

MODULE DESCRIPTION	:	This module deals with pulling out disassembling and checking automatic transmission.

SUGGESTED DURATION	:	40 hours

SUMMARY OF LEARNING OUTCOMES:

- At the completion of the module the trainees/students should be able to

LO1.	Dismount automatic transmission assembly

LO2.	Disassemble automatic transmission.

LO3.	Conduct automatic transmission parts analysis

LO4.	Replace automatic transmission parts

LO1.	DISMOUNT AUTOMATIC TRANSMISSION ASSEMBLY

ASSESSMENT CRITERIA:

- A/T fluid is drained thoroughly before transmission is removed

- A/T cables and linkages are disconnected

- Appropriate tools are used

CONTENTS:

- Procedure in draining fluid of automatic transmission

- Procedure in disconnecting and marking cable and linkages

- Types of oil used in automatic transmission.

CONDITIONS:

Students/trainees must be provided with the following:

- Car equipment with automatic transmission.

- Lifter, jack, stand

- Video CDs, tapes

- Mock-up of auto transmissions

METHODOLOGIES:

- Lecture

- Self-paced instruction

LO2.	DisASSEMBLE automatic transmission

ASSESSMENT CRITERIA:

- Disassemble parts are accurately marked/tagged

- Parts cleaned as per repair manual

CONTENTS:

- Correct procedure in dismounting automatic transmission

- Safety precaution in doing rise job.

CONDITIONS:

Students/trainees must be provided with the following:

- Car with auto transmission

- Lifter

- Basic hand tools

- Work bench

METHODOLOGIES:

- Lecture

- Self-paced instruction

LO3.	CONDUCT AUTOMATIC TRANSMISSION PARTS ANALYSIS

ASSESSMENT CRITERIA:

- A/T parts-hydraulic valve body, clutch and disc, bearing and planetary gears are checked/inspected for worn-out, crack, backlash and end play

- Inspection of parts is conducted

- Corresponding repair works is recommended

CONTENTS:

- Procedure in disassembling automatic transmission

- Cleaning of parts and components

CONDITIONS:

Students/trainees must be provided with the following:

- Mock up automatic transmission

- Working table

- Basic hand tools

- Specialized tools

- Cleaning pan

METHODOLOGIES:

- Lecture

- Self-paced instruction

LO4.	REPLACE AUTOMATIC TRANSMISSION PARTS

ASSESSMENT CRITERIA:

- Hydraulic valve body is replaced as per specification.

- Bearing/planetary gear is replaced as per specification.

- Clutch and disc are replaced as per specification.

CONTENTS:

- Procedure in replacing hydraulic valve body

- Procedure in replacing bearing/planetary gear

- Procedure in replacing clutch and disc

CONDITION:

Students/trainees must be provided with the following:

- Auto transmission

- Basic hand tools

- Specialized tools

- Dial gauge with magnetic stand

METHODOLOGIES:

- Lecture/demonstration

- Self-paced instruction

ASSESSMENT METHODS:

- Knowledge

- Practical

- Interview

- Direct observation

UNIT OF COMPETENCY	:	SERVICE AUTOMATIC TRANSMISSION

MODULE TITLE	:	ASSEMBLING AND TESTING AUTOMATIC TRANSMISSION

MODULE DESCRIPTION	:	This module deals with assembling procedures and testing procedures per specification and manual.

SUGGESTED DURATION	:	40 hours

SUMMARY OF LEARNING OUTCOMES:

At the completion of the module the trainees/students should be able to:

LO1.	Assemble automatic transmission

LO2.	Mount automatic transmission assembly

LO3.	Test automatic transmission

LO4.	Road test automatic transmission

LO1.	Assemble automatic transmission

ASSESSMENT CRITERIA:

- Assembling procedure is followed.

- Assembled parts are lubricated.

CONTENTS:

- Configuration of cable and linkages in automotive transmission and its adjustment point.

- Procedure in assembling automotive transmission

- Correct bolt and nut torque specification.

- A/T component repair and adjustment procedures (relevant to application)

- Equipment/material safety requirements

- Personal safety requirements

- Manual handling procedures

CONDITIONS:

Students/trainees must be provided with the following:

- Automotive transmission

- Manuals

- Hydraulic lifter/mechanical lifter

- Support stand

- Transmission jack

- Socket wrench

- Snap ring expander

- Snap ring compressor

- Micrometer

- Allen wrench

- Combination wrench

- Basic handtools

- A/T pressure gauge

- Torque wrench

- Dial indicator with stand

- Air compressor

- Hard rubber mallet

- Ballpeen hammer

METHODOLOGIES:

- Lecture

- Self-paced instruction

ASSESSMENT METHODS:

- Observation with questioning

- Demonstration with questioning

- Written examination

LO2.	Mount automatic transmission ASSEMBLY

ASSESSMENT CRITERIA:

- Bolts are tightened in accordance with manual torque specification, sequence and pattern

- Transmission is connected/installed

- Cables and linkages are adjusted

- A/T fluid is refilled

CONTENTS:

- Procedure in mounting automotive transmission

- Lubrication points of automotive transmission.

- Principles of lubrication.

- Equipment/material safety requirements

- Personal safety requirements

- Manual handling procedures

CONDITIONS:

Students/trainees must be provided with the following:

- Automotive transmission

- Manuals

- Hydraulic lifter/mechanical lifter

- Support stand

- Transmission jack

- Socket wrench

- Hard rubber mallet

METHODOLOGIES:

- Lecture

- Self-paced instruction

ASSESSMENT METHODS:

- Observation with questioning

- Demonstration with questioning

- Written examination

LO3.	Test automatic transmission

ASSESSMENT CRITERIA:

- Stall test and lag test are conducted

- Line pressure test is conducted

CONTENTS:

- Procedure in testing newly overhauled automotive transmission

- Component/unit test procedures and evaluation (relevant to application)

- Equipment/material safety requirements

- Personal safety requirements

- Manual handling procedures

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle equipped with automatic transmission

- Specialized testing equipment

- Manometer

METHODOLOGIES:

- Lecture/demonstration

- Self-paced instruction

ASSESSMENT METHODS:

- Observation with questioning

- Demonstration with questioning

- Written examination

LO4.	ROAD Test automatic transmission

ASSESSMENT CRITERIA:

- A/T fluid level, cable and linkages, engine operation are checked before road test

- Report of test is completely accomplished and corresponding recommendation is given

CONTENTS:

- Procedure in testing newly overhauled automotive transmission

- Equipment/material safety requirements

- Personal safety requirements

- Manual handling procedures

CONDITIONS:

Students/trainees must be provided with the following:

- Car equipment with automatic transmission

- Specialize testing equipment

- Manometer

METHODOLOGIES:

- Lecture/demonstration

- Self-paced instruction

ASSESSMENT METHODS:

- Observation with questioning

- Demonstration with questioning

- Written examination

UNIT OF COMPETENCY	:	PERFORM MAINTENANCE SERVICE CHECK-UP AND REPAIR TO AIRCONDITIONING SYSTEM

MODULE TITLE 	:	PERFORMING MAINTENANCE SERVICE CHECK-UP AND REPAIR TO AIRCONDITIONING SYSTEM

MODULE DESCRIPTOR 	:	This module identified the competence required to maintain automotive air-conditioning system and record operating condition

NOMINAL DURATION 	: 	40 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module, the trainee/student must be able to:

LO1.	Diagnose air-conditioning system

LO2.	Service auto air-conditioning system components

###### LO1.	Diagnose AIR-CONDITIONING System

ASSESSMENT CRITERIA:

- Diagnosis is in accordance with company standard operating procedures and manufacturer specification.

- Test is completed without causing damage to any workplace property or vehicle.

- Correct information is accessed /interpreted from appropriate manufacturer specification.

- System performance is tested and the appropriate air-conditioning service procedures are determined.

- Correct information is recorded on the service slip.

- Service of the system and component are carried out in accordance with vehicle/system manufacturer specification.

- Servicing is carried out according to industry regulations/guidelines, occupational health and safety procedure and the Philippine clear air act.

- System is tested and results are recorded in accordance with company policies and procedures.

CONTENTS:

- Testing procedure to auto air-conditioning system.

- Accomplishing service slip.

- Philippine clear air act.

- OH and S legislation.

- Measuring and testing procedures.

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle with AC system

- Manuals

- Aircon performance tester with thermometer

METHODOLOGIES:

- Traditional

- Dual training

- Self-paced instruction

ASSESSMENT METHODS:

- Observation with questioning

- Demonstration with questioning

# LO2.	SERVICE auto air-conditioning system components

ASSESSMENT CRITERIA:

- General servicing is completed without causing damage to any workplace property or air conditioning system component.

- Correct information is accessed and interpreted from appropriate manufacturer.

- Air conditioning system components are dismantled, reassembled and tested to manufacturer specification.

- Worn, damaged, deteriorated or faulty components are identified and replaced/repaired.

- System components are tested prior to placing into service and results are recorded in accordance with enterprise policies and procedures.

- Air-conditioning system components are overhauled according to industry regulation/guidelines/occupational health and safety procedure and company procedure/policies.

CONTENTS:

- Proper servicing of air conditioning system components.

- Appropriate use of manufacturer’s manual.

- Testing of air-conditioning component.

- Servicing procedure.

- Repair and replace air-conditioning components.

CONDITIONS:

Students/trainees must be provided with the following:

- Auto air conditioning component

- Service manual

- Fender cover

- Safety gloves

- Safety goggles

- VOM

METHODOLOGIES:

- Traditional/lecture and demonstration

- Dual training

- Distance learning

ASSESSMENT METHODS:

- Observation with questioning

- Demonstration with questioning

UNIT OF COMPETENCY	:	INSTALL AIR CONDITIONING SYSTEMS

MODULE TITLE	:	INSTALLING AIR CONDITIONING SYSTEMS

MODULE DESCRIPTOR 	:	This module identifies the competence required to install automotive air-conditioning system.

NOMINAL DURATION 	:	40 hours

PREREQUISITE	:	Basic knowledge in auto air-conditioning system

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module, the trainee/student must be able to:

LO1.	Fabricate brackets and air-conditioning pipings

LO2.	Install air conditioning systems

LO3.	Leak test air conditioning system

LO4.	Evacuate air-conditioning system

LO5.	Charge auto air-conditioning system

LO6.	Test air-conditioning system operation

###### LO1.	FABRICATE BRACKETS AND air-conditioning PIPINGS

ASSESSMENT CRITERIA:

- Brackets are fabricated and installed according to job requirements.

- Pipings are checked/replaced/installed.

CONTENTS:

- Fabrication of brackets and pipings.

- Safe handling of auto air conditioning system components.

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle with air conditioning system

- Nitrogen

- Gloves

- Oxy-acetylene welding outfit or equivalent

- Basic tools for air conditioning system

METHODOLOGIES:

- Traditional/lecture/demonstration

- Dual training

ASSESSMENT METHODS:

- Written and practical examination

- Interview

# LO2.	INSTALL air-conditioning systemS

ASSESSMENT CRITERIA:

- Automotive air conditioning system electrical wiring connections is carried out according to the required steps, techniques and procedures.

- Pressure valve is checked/replaced/Installed with care.

- Evaporator and condenser are checked/replaced/Installed with care.

- Expansion valve, thermostat, filter/sight glass are checked/replaced/installed with care.

- Compressor is checked/replaced/installed with care.

- Vehicle and equipment safety requirements are carried out according to step and procedures.

- Personal safety requirements are observed.

CONTENTS:

- Automotive air conditioning system electrical circuit diagram

- Procedures in connecting automotive air conditioning system electrical circuit.

- Components of auto air conditioning system.

- Procedures in checking/replacing/installing automotive air conditioning components.

- Safe handling of tools and equipment.

- PPS requirements.

CONDITIONS:

Students/trainees must be provided with the following:

- Wires

- Vehicle with air conditioning system

- Nitrogen

- System analyzer

- VOM/multitester

- Gloves

- Oxy-acetylene welding outfit or equivalent

- Basic tools for air conditioning system

METHODOLOGIES:

- Traditional/lecture and demonstration

- Dual training

- Distance learning

ASSESSMENT METHODS:

- Written and practical examination

- Interview

###### LO3.	LEAK Test air-conditioning system

ASSESSMENT CRITERIA:

- Tests are completed without causing damage to any automotive air conditioning system components.

- Correct information is accessed and interpreted from appropriate manufacturer’s specification.

- Tests are carried out to determine faults using appropriate tools and techniques.

CONTENTS:

- Test automotive air conditioning system components.

- Appropriate manufacturing specification.

- Test fault using appropriate tools and techniques.

CONDITIONS:

Students/trainees must be provided with the following:

- VOM

- Basic tools for air conditioning system

- Soap and water or leak tester

- Vehicle with air conditioning system.

METHODOLOGIES:

- Traditional

- Dual training

- Interview

ASSESSMENT METHODS:

- Written and practical examination

- Interview

###### LO4.	EVACUATE air-conditioning system

ASSESSMENT CRITERIA:

- Evaluation is completed using approved air conditioning vacuum equipment.

- Evacuation is performed according to required steps, techniques and procedures.

- Presence of moisture and positive pressure in the system is removed completely from the air-conditioning system.

CONTENTS:

- Procedures in evacuating automotive air conditioning system.

- Using appropriate tools and equipment.

- Using appropriate measuring instruments.

CONDITIONS:

Students/trainees must be provided with the following:

- Basic tools and equipment for servicing air conditioning system

- Vehicle with air conditioning system.

METHODOLOGIES:

- Traditional

- Dual training

- Interview

ASSESSMENT METHODS:

- Written and practical examination

- Interview

###### LO5.	CHARGE air-conditioning system

ASSESSMENT CRITERIA:

- Recharging is completed using approved air conditioning charging equipment.

- Recharging is performed according to required steps, techniques and procedures.

- Refrigerant used is applicable to the AC system and vehicle.

- Refrigerant leak testing is performed.

- Pressure reading of low side and high side is observed and checked.

- Amount of refrigerant charge is in accordance with manufacturer’s manual.

CONTENTS:

- Procedures in charging automotive air conditioning system.

- Procedures in leak testing air conditioning system.

- Using appropriate tools and equipment.

- Using appropriate measuring instruments.

CONDITIONS:

Students/trainees must be provided with the following:

- Basic tools and equipment for charging air conditioning system

- Vehicle with air conditioning system.

- Appropriate refrigerant for the system.

- Vehicle’s air-conditioner manual.

METHODOLOGIES:

- Traditional

- Dual training

- Interview

ASSESSMENT METHODS:

- Written and practical examination

- Interview

###### LO6.	TEST air-conditioning systemS OPERATION

ASSESSMENT CRITERIA:

- System performance test is conducted as per procedure.

- System components and controls are tested and checked/observed.

- Aircon cooling system performance/temperature and automatic intervals are checked and observed using a thermometer.

- Pressure reading of low side and high side is observed and checked.

- System performance results are within the manufacturer’s specifications.

CONTENTS:

- Procedures in testing automotive air conditioning system.

- Procedures in checking temperature and automatic intervals.

- Using appropriate measuring instruments.

- Reading low and high side pressure readings.

- Recording system performance.

CONDITIONS:

Students/trainees must be provided with the following:

- Measuring instruments/gauges

- Vehicle with air conditioning system.

- Vehicle’s air-conditioner manual.

METHODOLOGIES:

- Traditional

- Dual training

- Interview

ASSESSMENT METHODS:

- Written and practical examination

- Interview

UNIT OF COMPETENCY	:	SERVICE AIRCON COMPRESSOR AND ASSOCIATED COMPONENTS

MODULE TITLE	:	SERVICING AIRCON COMPRESSOR AND ASSOCIATED COMPONENTS

MODULE DESCRIPTOR 	:	This module identifies the competence required to service air-conditioning compressor and associated components.

NOMINAL DURATION 	:	52 hours

PREREQUISITE	:	Basic knowledge in automotive air-conditioning system

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module, the trainee/student must be able to:

LO1.  Diagnose air-conditioning compressor

LO2.	Pull down compressor assembly

LO3.	Disassemble compressor.

LO4.	Perform leak test of air-conditioning compressor

LO5.	Assemble compressor

LO6.	Install air-conditioning compressor

LO7.	Evacuate and recharge air-conditioning system

LO8.	Conduct compressor performance test

LO1.	DIAGNOSE AIR-CONDITIONING COMPRESSOR

ASSESSMENT CRITERIA:

- Pulley alignment is checked.

- Compression system is checked.

- Compressor gasket and o-ring is checked.

- Compressor magnetic clutch coil is checked.

- Service valves are checked.

- Compressor bearing are checked.

- Compressor electrical system is checked.

CONTENTS:

- Auto air conditioning system.

- Compressor lubrication system.

- Function and operation of compressor.

- Different types of compressor.

- Functions of magnetic clutch coil.

- Functions, types and operation of overload protector.

- Pulley alignment to the engine.

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle with air conditioning system

- VOM Multi tester

- Gloves

- Basic tools for air conditioning system

- Fender cover

- Rag

- Apron

- Soap and water

METHODOLOGIES:

- Interactive discussion

- Demonstration

- Self-paced instruction

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO2.	Pull down compressor assembly

ASSESSMENT CRITERIA:

- Timing mark on belt and pulley are placed.

- Holding bolts are loosened.

- Belt is loosened and removed from the compressor assembly.

- Fittings from discharge line and suction line are loosened and removed.

- Magnetic clutch connections are disconnected.

- Compressor assembly is removed from the engine.

CONTENTS:

- Different types of tools and their uses.

- Different types of belts.

- Different types of fittings

- Safety in handling compressor.

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle with air conditioning system

- Gloves

- Basic tools for air conditioning system

- Fender cover

- Marking pen

- Rag

- Apron

- Soap and water

METHODOLOGIES:

- Interactive discussion

- Demonstration

- Self-paced instruction

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO3.	Disassemble compressor

ASSESSMENT CRITERIA:

- Compressor motor oil is drained.

- Compressor is secured in a table vise or clamp.

- Special tools for disassembling compressors are identified.

- Magnetic clutch is removed.

- Compressor pulley is removed.

- Shaft seal is removed.

- Piston cover is removed.

- Piston and swash plate assembly are removed.

- Compressor shaft is removed.

- Control valves are removed.

- Parts of compressor are inspected and findings are recorded.

CONTENTS:

- Special service tools (SST), types and uses.

- Component parts of compressor.

- Operation of the compressor.

- Function of different parts of the compressor.

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle with air conditioning system

- Gloves

- Basic tools for air conditioning system

- Special service tools (SST)

- Rag

- Apron

- Gasoline

- Cleaning tray

- Soap and water

METHODOLOGIES:

- Interactive discussion

- Demonstration

- Self-paced instruction

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO4.	Perform leak test of AIR-CONDITIONING compressor

ASSESSMENT CRITERIA:

- Compressor is charged with refrigerant.

- Leak testing instrument is identified and used.

- Leak testing is performed.

- Component with leak is tightened or sealed.

CONTENTS:

- Refrigerant charging

- System leak test.

- Locate source of leak.

- Tighten or seal leaking components.

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle with air conditioning system

- Gloves

- Basic tools for air conditioning system

- Torque wrench

- Leak tester

- Rag

- Apron

- Soap and water

METHODOLOGIES:

- Interactive discussion

- Demonstration

- Self-paced instruction

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO5.	assemble compressor

ASSESSMENT CRITERIA:

- Compressor parts are cleaned, wiped with clean cloth and air-dried.

- Parts are assembled according to sequence.

- Gaskets and o-rings are replaced with new ones.

- Bolts and nuts are fixed and tightened

- Compressor oil is replaced.

- Compressor through the pulley is hand-turned for proper operation.

- Discharge and suction valves are checked for proper operation.

CONTENTS:

- Special service tools (SST), types and uses.

- Quantity and viscosity of compressor oils.

- Internal operation of the compressor.

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle with air conditioning system

- Gloves

- Basic tools for air conditioning system

- Special service tools (SST)

- Rag

- Apron

- Soap and water

METHODOLOGIES:

- Interactive discussion

- Demonstration

- Self-paced instruction

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO6.	Install AIR-CONDITIONING compressor

ASSESSMENT CRITERIA:

- Compressor is placed and aligned to engine.

- Pulley is aligned to the crankshaft pulley.

- Holding bolts are placed and tightened.

- Timing mark is set and aligned.

- Belt is installed.

CONTENTS:

- Installation of compressor to the engine

- Tightening of bolts with proper torque.

- Position of timing mark.

- Belt tension and alignment.

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle with air conditioning system

- Gloves

- Basic tools for air conditioning system

- Torque wrench

- Rag

- Apron

- Soap and water

METHODOLOGIES:

- Interactive discussion

- Demonstration

- Self-paced instruction

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO7.	Evacuate and recharge AIR-CONDITIONING system

ASSESSMENT CRITERIA:

- Gauge manifold is installed in the compressor.

- Refrigerant in the compressor is identified.

- Air is purged from the compressor.

- System is charged with refrigerant.

- Refrigerant charging point is sealed.

CONTENTS:

- Types of refrigerant.

- Compressor purging.

- Refrigerant charging.

- Leak testing.

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle with air conditioning system

- Gloves

- Basic tools for air conditioning system

- Torque wrench

- Leak tester

- Rag

- Apron

- Soap and water

METHODOLOGIES:

- Interactive discussion

- Demonstration

- Self-paced instruction

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO8.	Conduct compressor performance test

ASSESSMENT CRITERIA:

- Tightness of the belt is checked.

- Engine is checked.

- Engine is started.

- Car air-conditioner is operated (turned on).

- Performance of the compressor is recorded.

- Compressor is rechecked for leaks.

CONTENTS:

- Belt tension.

- Engine operation.

- Car air-conditioner operation.

CONDITIONS:

Students/trainees must be provided with the following:

- Vehicle with air conditioning system

- Gloves

- Leak tester

- Rag

- Apron

- Soap and water

METHODOLOGIES:

- Interactive discussion

- Demonstration

- Self-paced instruction

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

UNIT OF COMPETENCY	:	INSTALL LPG CONVERSION KIT

MODULE TITLE	:	INSTALLING LPG CONVERSION KIT

MODULE DESCRIPTOR 	:	This module covers the knowledge, skills and attitude required to install the component of LPG conversion kit to include the multi valve, tank, change over switch, vaporizer assembly, piping/hoses and safety gadgets

NOMINAL DURATION 	:	28 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module, the trainee/student must be able to:

LO1.	Perform vehicle test

LO2.	Mount and install conversion kit

LO3.	Install piping/hoses and safety features of the kit

LO1.	PERFORM VEHICLE TEST           (4 hours)

ASSESSMENT CRITERIA:

- Engine condition of the vehicle is checked based on standard operating procedure.

- Appropriate test instruments, tools and equipment are selected in accordance with job requirements.

- Occupational health and safety procedures are followed based on OH&S regulations.

- Findings on the test conducted are reported and recommendation is made to direct supervisor.

CONTENTS:

- Types of automotive engine, parts and functions

- Engine specifications

- Principles and concept of LPG fueled vehicle

- Occupational health and safety

- LPG-fuel conversion kit and accessories Installation and safety precautions

- Procedures in testing the vehicle

- Use of common automotive tools and equipment

- Measuring instruments

- Company workshop quality processes

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - LPG conversion kit - Battery load tester - Compression tester - Timing light - Hydraulic lifter - Air compressor | - Screwdriver - Hand drill - Pliers - Wrenches - Steel brush | - LPG fuel tank - Fuel piping lines and fittings - Solenoid valve - Safety valve - Emergency shut-off valve - Gloves - Fender cover - Rags - Apron - Hand cleaner | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO2.	MOUNT AND INSTALL CONVERSION KIT           (8 hours)

ASSESSMENT CRITERIA:

- Conversion kit is checked and tested before installation process is performed.

- Installation plan is checked in accordance with the job requirements.

- Installation information of LPG tank is accessed and interpreted based on manufacturer’s specifications

- Mounting and installation is performed in accordance with workplace procedure.

- Wiring installation is performed based on job requirements

CONTENTS:

- Checking and testing of conversion kit

- Procedure in mounting and installing conversion kit

- LPG-fuel conversion kit and accessories Installation and safety precautions

- Working plan for piping layout

- Types of LPG safety accessories

- Procedure in wiring installation

- Work, organization and planning processes

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - LPG conversion kit - Battery load tester - Compression tester - Timing light - Hydraulic lifter - Air compressor | - Screwdriver - Hand drill - Pliers - Wrenches - Steel brush | - LPG fuel tank - Fuel piping lines and fittings - Solenoid valve - Safety valve - Emergency shut-off valve - Gloves - Fender cover - Rags - Apron - Hand cleaner | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO3.  install piping / hoses and safety features of the kit    (16 hours)

ASSESSMENT CRITERIA:

- Piping/tubes, hoses and fittings are checked and tested before installation process is performed.

- Piping lay-out is performed at the front, underneath and rear of the vehicle

- Information is accessed and interpreted based on the piping installation specifications and safety rules

- Mounting and installation is performed based on procedures manual, safety handbook, manufacturer’s instruction and occupational health and safety (OH&S) requirements

CONTENTS:

- Principles and concept of mounting and installation of piping/hoses and safety features kit

- Interpretation of working plan for piping layout

- Checking and testing procedure on piping/tubes, hoses and fittings

- Piping lay-out procedure at the front, underneath and rear of the vehicle

- Safety rules in installing piping/hoses and safety features kit

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - LPG conversion kit - Battery load tester - Compression tester - Timing light - Hydraulic lifter - Air compressor | - Screwdriver - Hand drill - Pliers - Wrenches - Steel brush | - LPG fuel tank - Fuel piping lines and fittings - Solenoid valve - Safety valve - Emergency shut-off valve - Gloves - Fender cover - Rags - Apron - Hand cleaner | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

UNIT OF COMPETENCY	:	TEST AND ADJUST LPG CALIBRATION

MODULE TITLE	:	TESTING AND ADJUSTING LPG CALIBRATION

MODULE DESCRIPTOR 	:	This module covers the knowledge, skills and attitude required to test and adjusts Auto LPG calibration to include electrical system, ignition and starting, vaporizer or mixer.

NOMINAL DURATION 	:	26 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module, the trainee/student must be able to:

LO1.	Test electrical connections

LO2.	Adjust ignition system

LO3.	Adjust /tune-up vaporizer

LO4.	Check/test LPG fuel system

LO1.	TEST ELECTRICAL CONNECTIONS              (8 hours)

ASSESSMENT CRITERIA:

- Information is accessed and interpreted from appropriate manufacturer specifications.

- Tests are completed without causing damage to workplace property or vehicle.

- Tests are carried out to determine faults using appropriate tools and techniques.

- Faults are identified and preferred repair action is determined in accordance with workplace procedure.

CONTENTS:

- Interpretation of electrical connections

- Types of tools and testing instrument

- Electrical connection testing procedures and techniques

- Identification of fault

- Remedies and action on fault identified

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Supplies / Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - Multitester - Timing light - Hydraulic jack - Hydraulic lifter | - Screwdriver - Pliers - Wrenches - Feeler gauge | - Carburetor/EFI & SGI system - LPG fuel tank - Solenoid valve - Safety valve - Emergency shut-off valve - Ignition system components (sparkplug, contact points, rotor, distributor cap, ignition switch and high tension cables | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO2.	ADJUST IGNITION SYSTEM            (4 hours)

ASSESSMENT CRITERIA:

- Ignition system components are cleaned in accordance with job requirement

- Engine is started as per standard operating procedures.

- Ignition system is checked/ tested in accordance with workplace procedure.

- Ignition timing is adjusted based on engine performance.

- Ignition system/ components are repaired without causing damage to any other vehicle parts.

CONTENTS:

- Procedure in cleaning  ignition system components

- Operating procedures in starting an engine

- Procedure in checking and testing ignition system.

- Technique in adjusting ignition timing

- Procedures in repairing ignition system/ components

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Supplies / Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - Multitester - Timing light - Hydraulic jack - Hydraulic lifter | - Screwdriver - Pliers - Wrenches - Feeler gauge | - Ignition system components (sparkplug, contact points, rotor, distributor cap, ignition switch and high tension cables | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO3.	ADJUST / TUNE-UP VAPORIZER              (10 hours)

ASSESSMENT CRITERIA:

- Tests are carried out to tune the vaporizer or mixer using appropriate tools and techniques.

- Information is accessed and interpreted from appropriate manufacturer specifications.

- Tests are completed without causing damage to any workplace property or vehicle.

CONTENTS:

- Procedures and techniques in testing and tuning-out of vaporizer or mixer

- Types and classification of vaporizer system

- Interpretation of manufacturer’s  specifications

- Occupational health and safety

- Principles and concept in testing vaporizer or mixer

- Work, organization and planning processes

- Company workshop quality processes

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Supplies / Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - Multitester - Timing light - Hydraulic jack - Hydraulic lifter | - Screwdriver - Pliers - Wrenches - Feeler gauge | - Fuel filter and air cleaner - Carburetor/EFI & SGI system - Fuel piping lines and fittings - LPG fuel tank - Solenoid valve - Safety valve - Emergency shut-off valve - Ignition system components (sparkplug, contact points, rotor, distributor cap, ignition switch and high tension cables | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO4.	CHECK / TEST LPG FUEL SYSTEM       (4 hours)

ASSESSMENT CRITERIA:

- LPG fuel system components are checked according to the manual.

- Fuel piping lines are checked and tested for leaks based on workplace procedure.

- LPG fuel tank is checked in accordance with occupational health and safety procedures.

CONTENTS:

- Procedures and techniques in checking/testing LPG fuel system

- Procedures and techniques in checking and testing leaks

- Procedures in checking LPG tanks

- Work, organization and planning processes

- Company workshop quality processes

- Occupational health and safety procedure

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Supplies / Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - Multitester - Timing light - Hydraulic jack - Hydraulic lifter | - Screwdriver - Pliers - Wrenches - Feeler gauge | - Fuel filter and air cleaner - Carburetor/EFI & SGI system - Fuel piping lines and fittings - LPG fuel tank - Solenoid valve - Safety valve - Emergency shut-off valve | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

UNIT OF COMPETENCY	:	SERVICE AUTO LPG SYSTEM

MODULE TITLE	:	SERVICING AUTO LPG SYSTEM

MODULE DESCRIPTOR 	:	This module covers the knowledge, skills and attitude required to service Auto LPG system.

NOMINAL DURATION 	:	16 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module, the trainee/student must be able to:

LO1.	Service electrical system

LO2.	Service ignition components

LO3.	Service vaporizer/mixer

LO4.	Service LPG fuel system

LO1.	SERVICE ELECTRICAL SYSTEM        (4 hours)

ASSESSMENT CRITERIA:

- Servicing of electrical system such as starting/ ignition system and its component is performed as per manufacturer’s manual using specified tools and equipment.

- Servicing are completed without causing damage to  workplace property or vehicle

CONTENTS:

- Types of Ignition System, Carburetor, SGI and EFI

- Steps and procedures in servicing electrical system

- Automotive electrical system and its component

- Use of common automotive tools and equipment

- Company workshop quality processes

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Supplies / Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - Multitester - Timing light - Hydraulic jack - Hydraulic lifter | - Hand drill - Screwdriver - Pliers - Wrenches - Feeler gauge | - Carburetor/EFI & SGI system - Solenoid valve - Safety valve - Emergency shut-off valve - Ignition system components (sparkplug, contact points, rotor, distributor cap, ignition switch and high tension cables | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO2.	SERVICE IGNITION COMPONENTS       (4 hours)

ASSESSMENT CRITERIA:

- Servicing of spark plug, contact points, rotor, distributor cap, and ignition switch is performed as per manufacturer’s specifications/procedures.

- Engine is started as per standard operating procedure.

- Servicing of ignition coil resistance is performed based on engine performance.

- Servicing of ignition timing is performed in accordance with workplace procedure.

- Ignition system/components are repaired without causing damage to other vehicle parts.

CONTENTS:

- Parts and functions of ignition system and its components

- Servicing of spark plug, contact points, rotor, distributor cap, and ignition switch Engine specifications

- Occupational health and safety

- Technique in servicing ignition timing

- Procedure in repairing ignition system/components

- Company workshop quality processes (5S)

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Supplies / Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - Multitester - Timing light - Hydraulic jack - Hydraulic lifter | - Hand drill - Screwdriver - Pliers - Wrenches - Feeler gauge | - Ignition system components (sparkplug, contact points, rotor, distributor cap, ignition switch and high tension cables | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO3.	SERVICE VAPORIZER / MIXER       (4 hours)

ASSESSMENT CRITERIA:

- Servicing is carried out to tune the vaporizer or mixer using appropriate tools and techniques.

- Information is accessed and interpreted from appropriate manufacturer specifications.

- Servicing is completed without causing damage to workplace property or vehicle.

CONTENTS:

- Procedures and techniques in servicing vaporizer or mixer

- Interpretation of manufacturer’s  specifications

- Occupational health and safety

- Principles and concept in servicing vaporizer or mixer

- Company workshop quality processes

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Supplies / Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - Multitester - Timing light - Hydraulic jack - Hydraulic lifter | - Hand drill - Screwdriver - Pliers - Wrenches - Feeler gauge | - Fuel filter and air cleaner - Carburetor/EFI & SGI system - Fuel piping lines and fittings - LPG fuel tank - Solenoid valve - Safety valve - Emergency shut-off valve - Ignition system components (sparkplug, contact points, rotor, distributor cap, ignition switch and high tension cables | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO4.	SERVICE LPG FUEL SYSTEM          (4 hours)

ASSESSMENT CRITERIA:

- LPG fuel system components are serviced according to manual and specifications.

- Fuel filter is replaced according to manufacturer’s instructions, as needed.

- Servicing is completed without causing damage to any workplace property or vehicle.

CONTENTS:

- Principles and concept in servicing LPG fuel system

- Procedures and techniques in servicing LPG fuel system

- Interpretation of manufacturer’s  specifications

- Occupational health and safety procedures

- Company workshop quality processes

- Steps in replacing fuel filter

- LPG-fuel conversion kit, accessories installation and safety precautions

- Multi-Valve, vaporizer, tank and change-over switch parts and nomenclature

- Installation procedure for valves, vaporizer change over switch, piping and  safety gadgets

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Supplies / Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - Multitester - Timing light - Hydraulic jack - Hydraulic lifter | - Hand drill - Screwdriver - Pliers - Wrenches - Feeler gauge | - Carburetor/EFI & SGI system - Fuel piping lines and fittings - LPG fuel tank - Solenoid valve - Safety valve - Emergency shut-off valve | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

UNIT OF COMPETENCY	:	REMOVE AND REPLACE AUTOMOTIVE ENGINE AND ENGINE-RELATED SYSTEMS

MODULE TITLE	:	REMOVING AND REPLACING AUTOMOTIVE ENGINE AND ENGINE-RELATED SYSTEMS

MODULE DESCRIPTOR 	:	This module covers the knowledge, skills and attitude required to remove and replace auto engine and engine-related system for repowering the vehicle.

NOMINAL DURATION 	:	30 hours

SUMMARY OF LEARNING OUTCOMES:

Upon completion of this module, the trainee/student must be able to:

LO1.	Prepare for work

LO2.	Remove engine and/ or related systems

LO3.	Replace engine and/ or related systems

LO4.	Clean up work area

LO1.	PREPARE FOR WORK                (4 hours)

ASSESSMENT CRITERIA:

- Work instructions are used to determine requirements, including methods, materials and equipment

- Job specifications are read and interpreted based on manufacturer’s specification

- Occupational Health and Safety (OH & S) requirements are observed throughout the work

- Replacement engine and related mechanical units/assemblies and materials for work are selected, inspected and tested

- Tools and equipment (including measuring/ test instruments) are identified and checked for safe operation

- Procedures are determined to minimize wastage and energy efficiency based on job requirement

CONTENTS:

- Interpretation of work instructions and job specifications

- Engine specifications

- Occupational health and safety regulations

- Use of common automotive tools and equipment

- Selection, inspection and testing of replacement engine and other related systems

- Types and uses of fuel or hazardous materials

- Measuring instruments

- Work, organization and planning processes

- Company workshop quality processes

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - Hydraulic jack - Hydraulic lifter - Chain block | - Hand drill - Screwdriver - Pliers - Wrenches | - Gloves - Fender cover - Rags - Apron - Hand cleaner - Spare parts - Lubricants - Cleaning materials - Fuel | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO2.	REMOVE ENGINE AND/OR RELATED SYSTEMS       (8 hours)

ASSESSMENT CRITERIA:

- Engines and other related systems are removed using appropriate methods, tools and equipment in accordance with workplace procedure.

- Engine and other related systems removal is completed without causing damage to component, system or injury to self or others.

- Task is carried out according to workplace procedure and occupational health and safety regulations.

CONTENTS:

- Types of automotive engine and other related system its parts and functions

- Procedures in removing and replacing engine and other related systems

- Occupational health and safety regulation

- Use of common automotive tools and equipment

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - Hydraulic jack - Hydraulic lifter - Chain block | - Hand drill - Screwdriver - Pliers - Wrenches | - Gloves - Fender cover - Rags - Apron - Hand cleaner - Spare parts - Lubricants - Cleaning materials - Fuel | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO3.	REPLACE ENGINE AND/OR RELATED SYSTEMS        (16 hours)

ASSESSMENT CRITERIA:

- Engines and other related systems are replaced with recommended units using appropriate methods, tools and equipment

- Replacement  of engines and other related systems is completed without causing damage to component or system or injury to self or others

- Inspection, testing and final adjustments is conducted according to company/manufacturer’s procedures

- Task is carried out according to workplace procedure and occupational health and safety regulations

CONTENTS:

- Procedure in replacing automotive engine and other related systems

- Techniques in replacing  engines and other related systems

- Steps and procedures in inspecting, testing and final adjustments

- Occupational health and safety regulations

- Use of common automotive tools and equipment

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - Hydraulic jack - Hydraulic lifter - Chain block | - Hand drill - Screwdriver - Pliers - Wrenches | - Gloves - Fender cover - Rags - Apron - Hand cleaner - Spare parts - Lubricants - Cleaning materials - Fuel | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

LO4.	CLEAN UP WORK AREA        (2 hours)

ASSESSMENT CRITERIA:

- Materials that can be reused is collected and stored in accordance with workplace procedure.

- Waste and scrap is removed/ disposed-off following workplace and environmental procedures.

- Equipment and work area are cleaned and inspected for serviceable condition in accordance with workplace procedures.

- Unserviceable equipment is identified, tagged and faults identified in accordance with workplace procedures.

- Tools and equipment, supplies and materials and accessories are stored in accordance with workplace procedures.

CONTENTS:

- Recycling and storing of materials

- Proper waste disposal

- Proper house keeping (5S) Storing , safekeeping of tools materials and equipment

- Occupational health and safety regulation

CONDITIONS:  Students/trainees must be provided with the following:

| Equipment | Tools | Materials | Manuals |
| --- | --- | --- | --- |
| - Vehicle - Hydraulic jack - Hydraulic lifter - Chain block | - Hand drill - Screwdriver - Pliers - Wrenches | - Gloves - Fender cover - Rags - Apron - Hand cleaner - Spare parts - Lubricants - Cleaning materials - Fuel | - Philippine National Standards (PNS 115) - Manufacturer specification manual - Maintenance procedure manual - Periodic maintenance manual - Service manual - Parts Checklist - LPG Conversion Kit Manual |

METHODOLOGIES:

- Lecture/discussion

- Demonstration

- Observation

ASSESSMENT METHODS:

- Interview

- Written examination

- Practical demonstration

- Direct observation

# What is Competency-Based Curriculum (CBC)

- A competency-based curriculum is a framework or guide for the subsequent detailed development of competencies, associated methodologies, training and assessment resources.

- The CBC specifies the outcomes which are consistent with the requirements of the workplace as agreed through the industry or community consultations.

- CBC can be developed immediately when competency standards exist.

- When competency standards do not exist, curriculum developers need to clearly define the learning outcomes to be attained. The standard of performance required must be appropriate to industry and occupational needs through the industry/enterprise or specified client group consultations.

These materials are available in both printed and electronic copies.

For more information please contact:

Technical Education and Skills Development Authority (TESDA)

Telephone Nos.: 893-8281, 817-4076 to 82 loc. 611, 630, 631 and 635

or visit our website: www.tesda.gov.ph or the TESDA Regional or Provincial Office nearest you.